UltraSound Lowlevel ToolKit

Revision 2.01

20 May 1993

Advanced Gravis

101-3750 North Fraser Way

Burnaby, British Columbia V5J 5E9

FAX (604)-431-5155

Forte Technologies

1555 East Henrietta Rd.

Rochester, N.Y. 14526

FAX (716)-292-6353

Advanced Gravis and Forte Technologies

Low Level Toolkit/Source for UltraSound

NOTICE

The information contained in this manual is believed  to  be

correct.  The manual is subject to change without notice and

does not represent a commitment on  the  part  of  FORTE  or

Advanced Gravis.

Neither FORTE nor Advanced Gravis make  a  warranty  of  any

kind with  regard  to  this  material,  including,  but  not

limited to, the implied warranties  of  merchantability  and

fitness  for  a  particular  purpose.   Neither  FORTE   nor

Advanced Gravis shall be liable for errors contained  herein

or for incidental or  consequential  damages  in  connection

with the furnishing, performance or use of this material.

This document  contains  proprietary  information  which  is

protected  by  copyright.   This  manual  is  Copyright  (C)

1992,1993 by FORTE and  Advanced  Gravis.   All  rights  are

reserved.  No part  of  this  document  may  be  reproduced,

transmitted, transcribed, stored in a retrieval  system,  or

translated into any human or computer language, in any  form

or by any means; electronic, mechanical, magnetic,  optical,

chemical, manual or otherwise, without the expressed written

permission of FORTE and Advanced Gravis.

Any copying, duplication, selling, or otherwise distributing

the program or support files described in this manual, other

than for the limited purposes of system backup  and  loading

the program into the  computer  as  part  of  executing  the

program, is a violation of the  software  license  agreement

and the law.  Willful violation of the copyright law of  the

United States can result  in  statutory  damages  of  up  to

$50,000  in  addition  to  actual  damages,  plus   criminal

penalties of imprisonment  for  up  to  one  year  and/or  a

$10,000 fine.

p                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 1

Chapter 1 - General Information

1.1  Features

- Jumper selectable base port address.

- Software selectable IRQ vectors and DMA channels.

- XT and AT compatibility.

- 8 or 16 bit playback.  Stereo and Monophonic.

- 8 bit recording.  Stereo or Monophonic.

- Playback and recording rates up to 44.1 kHz.

- 32 voice playback.  All voices mixed on board.

- Separate recording channel for simultaneous playback

and recording.

- Each voice has its own volume settings, volume enveloping,

playback rate and pan (balance) position.

- Both line level and amplified outputs.

- MIDI on-board.

- Gravis Eliminator joystick interface with jumper

enable/disable.

- 256K DRAM on board for waveforms.  Expandable to 1 MEG

- Right/Left panning (balance) on a per voice basis.

- Stereo microphone input with automatic level control.

- Line level input.

- CD ROM Drive audio input.

- Right/Left mini-jacks for line & amplified outputs and

Mic and Line inputs.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 2

Chapter 1 - General Information

1.2  MIDI Interface Functional Description

The  MIDI  101   interface   consists   of   standard   UART

functionality - Motorola MC68C50.  An interrupt to the PC is

generated for each byte of  data  received  or  transmitted.

This hardware is independent of any of the  other  hardware.

This circuitry is also included in the GF1.  External to the

GF1 is an optical isolator that is used on the serial  input

data and an open collector  driver  that  is  used  for  the

serial output.  In addition, external logic is  included  on

board to loopback transmit data to the  receive  data  under

software control.  The serial interface will  have  a  fixed

configuration with no programmable options as in the MC6850.

A control  register  is  used  to  enable  and  disable  the

interrupt generation logic.  A status register  is  used  to

determine  if  the   transmit   or   receive   register   is

interrupting.  A read or write to the data  register  clears

the  interrupt  status.   The  following  are  the  hardware

specifications:

31.25 KHZ +- 1%

asynchronous

1 start bit

8 data bits

1 stop bit

The MIDI signals are available on the  15  pin  D  connector

used  for  the  Joy  Stick.   An  external  cable   assembly

containing the optical isolator and driver  is  required  to

use MIDI.

1.3  JoyStick Interface

The joystick interface is an Eliminator  Joystick  interface

designed  by  Advanced  Gravis.   The   joystick   interface

consists of an eight bit register.  When  written  to,  four

flip-flops are reset and comparator inputs (LM339) begin  to

charge up based  on  the  position  of  the  joystick.   The

comparator threshold is setup  in  the  GF1.   Crossing  the

threshold of the comparators  cause  the  flip-flops  to  be

preset and the capacitor to be  discharged.   Reads  of  the

register return the state of four digital inputs (internally

pulled up) and the state of the  flip-flops.   The  rate  of

discharge of the capacitors has a minimum time constant of 1

us.  A jumper on board is used  to  enable  or  disable  the

joystick.

j                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 3

Chapter 1 - General Information

1.4  GF1 - 32 Voice Sound Synthesizer

The method of  sound  synthesis  is  Wave  Table  Synthesis.

Either  sampled  data  from  actual  instruments  or   other

synthesized  digital  audio  is  stored  in  memory  on  the

Ultrasound Board.  The GF1 is setup to  playback  relatively

short digital audio samples  and  produce  continuous  sound

exactly reproducing the original instrument.  The 32  voices

are independent and can concurrently be producing  different

sounds which are mixed into either the left or right channel

output.  Circuitry in the GF1 can be programmed  to  perform

the following audio processing functions  independently  for

each voice:

- Frequency Shifting to produce different notes of the  same

instrument.

- Amplitude Modulation to produce note  enveloping  (attack,

decay, sustain, release), overall volume control or  special

effects such as LFO (low frequency amplitude modulation)

- Panning the voice from left to right channel outputs.

This method of Audio synthesis is the same  method  used  in

expensive keyboards.

Continuous digital audio recordings can  be  played  through

the GF1 by using one voice per digital audio track.  The GF1

is compatible with 8 and 16 bit data, stereo or mono, signed

or unsigned data, and 8 or 16 bit DMA channels.

1.5  Theory of Operation

This section describes the theory behind  the  operation  of

the GF1.

The GF1 is basically a pipeline  processor.   It  constantly

loops from voice #0 to the end of the active  voices.   (How

to define then number of  active  voices  is  shown  later).

Every  1.6  microseconds,  the  GF1  performs  a  series  of

operations on a particular voice.  The  more  active  voices

there  are,  the  longer  it  takes  between  each  time   a

particular voice is serviced.  This puts a limit on the rate

at which playback can occur.  14 active voices will allow  a

maximum of 44.1 kHz playback.  28 voices will allow 22  kHz.

Faster  rates  can  be  achieved  by  making  the  frequency

constant greater than 1.  This will cause the  GF1  to  skip

some data bytes to play  a  sample  back  at  the  requested

frequency.  This is not generally a problem, but could cause

some distortion or aliasing.

q                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 4

Chapter 1 - General Information

The formula for calculating the playback rate is:

Frequency divisor table.

44100   14 active voices */

41160   15 active voices */

38587   16 active voices */

36317   17 active voices */

34300   18 active voices */

32494   19 active voices */

30870   20 active voices */

29400   21 active voices */

28063   22 active voices */

26843   23 active voices */

25725   24 active voices */

24696   25 active voices */

23746   26 active voices */

22866   27 active voices */

22050   28 active voices */

21289   29 active voices */

20580   30 active voices */

19916   31 active voices */

19293   32 active voices */

This table is calculated by knowing that  14  active  voices

will give exactly 44.1 khz playback.  Therefore:

1,000,000 / (X * 14) = 44100

X = 1.619695497

Once that is known, the frequency divisor is calculated by:

1,000,000 / (1.619695497 * # of active voices)

The lowlevel code pre-calculates this table (see  vocfreq.c)

so that floating point arithmetic doesn't need to  be  done.

To  calculate  a  FC  (frequency  counter)  for  any   given

frequency with a particular  #  of  active  voices,  run  it

through this formula:

fc = (unsigned int)(((speed_khz<<9L)+(divisor>>1L))/divisor);

fc = fc << 1;

(The last left shift is needed because the  FC  is  in  bits

15-1.  Bit 0 is not used).

This is then put in the frequency control register for  that

particular voice.  If the mantissa portion of the FC  is  1,

then each time around the loop, the GF1 uses each data point

to  play.   If  there  is  a  fractional  portion,  the  GF1

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 5

Chapter 1 - General Information

interpolates the actual data  to  play  from  the  two  data

points that it  is  between.   This  makes  the  sound  much

'smoother', since the GF1 will create points in between  the

actual data points.  For example, assume an 8 bit  recording

at 22 khz and 14 active  voices  .   The  frequency  control

register is set up to 1/2 (exponent = 256).  This means that

every  time  around  the  loop,   that   particular   voices

accumulator is adjusted by  1/2.   So  the  first  time  the

accumulator is 0 and data point 0 is used.  The second  time

around the  loop,  the  accumulator  is  0.5.   Since  there

obviously is no DRAM location 0.5, the GF1 interpolates what

the data would be by looking at location 0  and  location  1

and taking the appropriate ratio from each.  In  this  case,

it picks a point half-way between the two.  If the recording

rate were 11khz, it would take 25% from location 0  and  75%

from location 1 the first time thru the loop.  The next time

it would take 50% from each.  The next time  it  would  take

25% from location 0 and 75% from location 1.  The forth time

thru it uses 100% of location 1.

The interpolation is done to a resolution of 16  bits,  even

for 8 bit playback.  This interpolation has  the  effect  of

making an 8 bit recording sound better when played  back  on

the GF1 than on a standard 8 bit card.

Remember  that  the  GF1  works  on  a   voice   every   1.6

microseconds.  This means that the fewer voices, the  faster

each voice gets updated.   The  frequency  control  register

setting for the voice MUST take this into account.   The  FC

must get  smaller  if  the  number  of  active  voices  gets

smaller.  This will increase the number  of  points  created

between the actual data points  so  the  perceived  playback

speed remains the same.

NOTE: The volume enveloping uses  the  same  principle  when

ramping up and down.

1.6  Hardware Volume Ramping

The Ultrasound has built-in volume ramping to facilitate the

implementation  of  the   attack   decay   sustain   release

envelopes.  A section of the envelope can be programmed such

that the PC does not need to be burdened with  the  task  of

changing each volume at specified intervals.  At the end  of

that particular section, an IRQ can be generated so that the

next section can be programmed in.  This continues until the

entire envelope has  been  completed.   The  start  and  end

points as well as the increment rate are fully programmable.

The register definitions are:

t                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 6

Chapter 1 - General Information

Current Volume (9,89)    EEEEMMMMMMMM    (Bits 15-4)

Volume Start (7,87)      EEEEMMMM        (Bits 7-0)

Volume End (8,88)        EEEEMMMM        (Bits 7-0)

Volume Incr (6,86)       RRMMMMMM        (Bits 7-0)

Once the current volume, start and end volumes are programmed,

the only thing left is to set up the volume increment register.

This register determines how fast the ramp takes place and with

what granularity. The finer the granularity, the smoother (but

slower) the ramp. The increment register has 2 fields. The first

is the amount added to (or subtracted from) the current volume

to get to the next one. These are the low 6 bits and can range

from 1 to 63. A 1 is a long, slow ramp compared to a 63. The

upper 2 bits determine how often the increment is applied to the

current volume. The rate bits are defined as:

Rate Bits       Volume Update Rate

00                FUR       (FUR = 1/(1.6*#active voices))

01                FUR/8

10                FUR/64

11                FUR/512

Each rate increment is 8 times  longer  than  the  preceding

one.  This means that the value to  store  for  the  fastest

possible ramp is 0x1f (63), and the value  for  the  slowest

possible ramp is 0xc1 (193).  The approximate  times  for  a

full scale volume ramp (0-4095) are:

Rate    Vol Inc    14 Voices     32 Voices

0        63         1.4 ms        3.3 ms

0        1         91.7 ms      209.7 ms

1        63        11.5 ms       26.2 ms

1        1        733.8 ms        1.7 sec

2        63        91.8 ms      209.7 ms

3        1          5.9 sec      13.4 sec

3        63       734.0 ms        1.7 sec

3        1         47.0 sec     107.3 sec

Note that these times are for full scale ramping.  Since the

volume ramps usually go between points in between the rails,

the actual ramp times will be much smaller.

The volume ramping can be very useful for things other  than

the enveloping.  Since there are only 16  pan  positions,  a

balance sweep from right to left may  produce  clicks  since

there is such a large jump between pan positions.   You  can

get a very smooth balance sweep using 2  voices  and  volume

ramping.  Just set one voice up to the right, one up to  the

p                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 7

Chapter 1 - General Information

left and ramp one down from volume X to  zero  at  the  same

rate as you ramp the other from 0 up to volume X.

It can also be  used  to  remove  clicks  and  pops  at  the

beginning and end of digital samples.  By setting up  fairly

fast rates when a sample start or ends  and  ramping  up  or

down appropriately, any pop created by a  sudden  change  in

the DAC value will be summed in at such  a  low  volume,  it

will never be heard.

1.7  Tips on click removal.

There are several ways that clicks and pops may be  produced

on the Ultrasound.  Depending on what causes it, it  can  be

very tricky to prevent.

1.7.1  During Reset

The  simplest  click  to  create  and   remove   is   during

initialization.  When the GF1 is reset, a  pop  may  happen.

To  prevent  this,  only  reset  the  GF1  with  the  output

disabled.

1.7.2  During balance sweep

Since there is only 16 pan positions,  it  is  quite  likely

that  you  will  get  pops  when  changing  pan   positions.

Particularly if it is  done  at  a  fairly  high  rate.   To

overcome this, use 2 voices and  volume  ramping.   Set  the

balance positions on both voices, and ramp one down  at  the

same rate the other ramps up.  The result is a  very  smooth

balance sweep.

1.7.3  When a voice starts

A click can happen  when  the  output  of  the  DAC  changes

suddenly.  This is the most frequent type  of  click,  since

there may not be a way to control the data values the  voice

is trying to play.  The way to avoid the click is to set  up

a volume ramp that starts at a low volume  when  the  sample

starts and ramps up to an  audible  volume.   That  way  the

click will be summed in at such a low volume  that  it  will

not contribute much to the final output DAC value.

w                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 8

Chapter 1 - General Information

1.7.4  When a voice ends

This usually happens because a voice  stops  abruptly.   The

solution to this is to set up a fast volume ramp  that  will

take the voice from  its  current  volume  down  to  0  very

quickly.

1.7.5  General

It is necessary to remember that all voices are being summed

in to the final output, even if they are not running.   This

means that whatever data value that the voice is pointing at

is contributing to the summation.  It is  important  that  a

voice be pointed to a known value at a known location  after

it is stopped so that some control is  kept  over  it.   For

instance, if a  voice  were  left  at  where  ever  the  end

position was for the last time it played, a pop could  occur

if new data were either DMA'ed or poked over the top of  it.

It is recommended that a voice  be  pointed  to  a  location

containing a 0 and that its volume be set to  0.   Now  that

voice will have no contribution to the output.

A possibly useful  side-effect  of  this  summation  process

could be called direct DAC output.  This can be accomplished

by setting a voice up to point to a specific location with a

given volume and poking in the  data  bytes  at  a  constant

rate.  It will not sound as good as letting the GF1 play the

sound because no interpolation or oversampling  will  occur.

However, it could be useful for some applications.

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound              Page 9

Chapter 2 - Hardware Information

2.1  I/O Port Map

The following describes I/O address map used on  the  board.

The 'X' is defined by the jumper settings on the  UltraSound

and should match that specified in the ULTRASND  environment

variable.

INTERFACE                         I/O,MEM,  R,W       ADDRESS

INT,DMA             HEX

------------------------------------------------------------

MIDI Interface:

Control                       I/O       W         3X0

Status                        I/O       R         3X0

Transmit Data                 I/O       W         3X1

Receive Data                  I/O       R         3X1

Joystick Interface:

Trigger Timer                 I/O       W         201

Read Data                     I/O       R         201

GF1 Synthesizer:

GF1 Page Register             I/O       R/W       3X2

GF1/Global Register Select    I/O       R/W       3X3

GF1/Global Data Low Byte      I/O       R,W       3X4

GF1/Global Data High Byte     I/O       R/W       3X5

IRQ Status Register 1=ACTIVE  I/O       R         2X6

Timer Control Reg             I/O       R/W       2X8

Timer Data                    I/O       W         2X9

DRAM                          I/O       R,W       3X7

DRAM                          DMA       R,W       1,3,5,6,7

Record Digital Audio          DMA       R         1,3,5,6,7

BOARD ONLY

Mix Control register          I/O       W         2X0

IRQ  control register         I/O       W         2XB

(2X0- bit 6 = 1)

DMA control register          I/O       W         2XB

(2X0- bit 6 = 0)

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 10

Chapter 2 - Hardware Information

At powerup the board will have operational Joystick and MIDI

interfaces.  This will allow their direct use with  existing

software.  The GF1  ASIC  will  power  up  with  all  voices

disabled not requiring a software initialization.  This will

help eliminate noise at powerup and allow the  Joystick  and

MIDI interfaces to be used by  existing  applications.   The

IRQ  control  register  MUST  be  set  up  before  the  MIDI

interface can generate an IRQ.  This is done in ultrinit.exe

and when an application sets up the latches to the  ULTRASND

parameters.

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 11

Chapter 2 - Hardware Information

2.2  MIDI Control Port - 3X0

Here are the bit definitions for the midi control byte.

It is located at the 3X0 hex and is write only.

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- 1 \ Master reset (when set)

|   |   |   |   |   |   +-------- 1 /

|   |   |   |   |   +------------ Reserved

|   |   |   |   +---------------- Reserved

|   |   |   +-------------------- Reserved

|   |   +------------------------ 1 \ xmit IRQ enabled

|   +---------------------------- 0 /

+-------------------------------- 1 = Receive IRQ enabled

Bit 0 & 1 will cause a master reset when toggled high and

then low. They must be left low when using port. This will

normally cause a transmit buffer empty irq.

U                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 12

Chapter 2 - Hardware Information

2.3  MIDI Status Port - 3X0

Here are the bit definitions for the midi status byte

It is located at the 3X0 hex and is read only.

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Receive reg. full

|   |   |   |   |   |   +-------- Transmit reg. empty

|   |   |   |   |   +------------ Reserved

|   |   |   |   +---------------- Reserved

|   |   |   +-------------------- Framing Error

|   |   +------------------------ Overrun error

|   +---------------------------- Reserved

+-------------------------------- Interrupt pending

The midi control behaves identically to a 6850 UART.

2.4  MIDI Data Port - 3X1

The transmit and receive registers are at 3X1 hex and

are 8 bits wide.

r                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 13

Chapter 2 - Hardware Information

2.5  Page Register - 3X2

This could also be called the voice select  register.   This

register is used to specify which voice's registers you want

to read/write.  This value can range from 0 to the number of

active  voices  specified  (13-31).   Once  this  has   been

specified, you may select the specific register within  that

voice.  Be careful that irqs are off during  the  time  that

the page and select registers are being modified.  This will

prevent the foreground from selecting a voice and having the

background change it in the background.

2.6  Select Register - 3X3

2.6.1  Global Registers

These are the global registers.   These  registers  are  not

voice-specific.

Address     Mode    Width   Description

41        R/W       8     DRAM DMA Control

42        W        16     DMA Start Address

43        W        16     DRAM I/O Address (LOW)

44        W         8     DRAM I/O Address (HIGH)

45        R/W       8     Timer Control

46        W         8     Timer 1 Count

47        W         8     Timer 2 Count

48        W         8     Sampling Frequency

49        R/W       8     Sampling Control

4B        W         8     Joystick trim DAC

4C        R/W       8     RESET

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 14

Chapter 2 - Hardware Information

2.6.1.1  DRAM DMA Control Register - (41)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Enable DMA (1=go)

|   |   |   |   |   |   +-------- DMA direction (1=read)

|   |   |   |   |   +------------ DMA channel width

|   |   |   |   +---------------- \

|   |   |   +-------------------- / DMA Rate divider

|   |   +------------------------ DMA IRQ Enable

|   +---------------------------- (R) DMA IRQ Pending

|                                 (W) DATA SIZE (0=8bit,1=16bit)

+-------------------------------- Invert MSB (write only)

Bit 0 - Enable the DMA channel. The GF1 will begin sending DMA ACK

protocol. If PC DMA controller is programmed, data will begin

being transferred. If not, data will move as soon as it is

programmed.

Bit 1 - DMA transfer direction. Read is taking data OUT of the

UltraSound. Write sends data to it.

Bit 2 - 0 = if DMA channel is an 8 bit channel (0-3).

1 = If it is a 16 bit channel (4-7)

Note: This is INDEPENDENT of the data size....

Bit 3,4 DMA Rate divisor. The Maximum rate is approx 650 khz.

00 = divide by 1

01 = divide by 2

10 = divide by 3

11 = divide by 4

Bit 5 - DMA terminal count interrupt enable

Bit 6 - Read - DMA terminal count IRQ pending

Write - Data size 0 = 8 Bit data

1 = 16 bit data

Note: Data size is independent of channel size

Bit 7 - 1 = Invert High bit to flip data to twos complement form.

Note: This flips bit 7 for 8 bit data and bit 15 for

16 bit data.

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 15

Chapter 2 - Hardware Information

2.6.1.2  DMA Start Address - (42)

Bits 15-0 are Address lines 19-4.

This register defines  where  the  DMA  will  transfer  data

to/from.  Since only the upper 16 address bits are used  and

the lower 4 bits are set to 0, a DMA transfer MUST begin  on

an 16 byte boundary for an 8 bit DMA channel (0-3).  If a 16

bit DMA channel is being used, the transfer MUST being on  a

32 byte boundary.   An  additional  address  translation  is

necessary if a 16 bit DMA channel is used.  Here  is  the  C

code to do the translation.  (See function convert_to_16()).

unsigned long  address;

unsigned long  hold_address;

hold_address = address;

/* Convert to 16 translated address. */

address = address >> 1;

/* Zero out bit 17. */

address &= 0x0001ffffL;

/* Reset bits 18 and 19. */

address |= (hold_address & 0x000c0000L);

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 16

Chapter 2 - Hardware Information

2.6.1.3  DRAM I/O Address (43,44)

These 2 registers allow you to specify an  address  to  peek

and poke directly into UltraSound DRAM.  Register 43 is  the

lower 16 address lines.  Register 44 is the upper 4  address

lines.  (bits 0-3).  Read or write to register 3X7 to get at

the address location.

2.6.1.4  Timer Control - (45)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Unused

|   |   |   |   |   |   +-------- Unused

|   |   |   |   |   +------------ Enable Timer 1 IRQ

|   |   |   |   +---------------- Enable Timer 2 IRQ

|   |   |   +-------------------- Unused

|   |   +------------------------ Unused

|   +---------------------------- Unused

+-------------------------------- Unused

2.6.1.5  Timer 1&2 Count - (46,47)

These counts are loaded by the  application  and  then  they

will count up to FF and generate and IRQ.   Timer  1  has  a

granularity of 80 microsecs and Timer 2 has a granularity of

320 microsecs.

2.6.1.6  Sampling Frequency - (48)

The formula for calculating this value is:

rate = 9878400/(16*(FREQ+2))

f                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 17

Chapter 2 - Hardware Information

2.6.1.7  Sampling Control Register - (49)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Start sampling

|   |   |   |   |   |   +-------- Mode (0=mono, 1=stereo)

|   |   |   |   |   +------------ DMA width (0=8bit,1=16bit)

|   |   |   |   +---------------- Unused

|   |   |   +-------------------- Unused

|   |   +------------------------ DMA IRQ enable

|   +---------------------------- (Read) DMA IRQ pending

+-------------------------------- Invert MSB

Bit 0 - If PC DMA controller is programmed, it will begin

sampling as soon as this is enabled.

Bit 1 - 0 = mono, 1 = stereo. In stereo mode, the order of

the data bytes is left is first, and right is second.

If a 16 bit data channel is used, the left is in the

lower byte.

Bit 2 - DMA Channel width (0=8bit, 1=16bit)

Bit 5 - Enable DMA terminal count IRQ

Bit 6 - DMA terminal count IRQ pending

Bit 7 - Flip bit 7 to get non-twos compliment data

v                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 18

Chapter 2 - Hardware Information

2.6.1.8  Joystick Trim DAC - (4B)

This register is initialized to 4.3 volts (value = 29).   It

only needs to  be  modified  to  account  for  faster/slower

machines.  A utility is provided (ultrajoy) that  sets  this

up.  There should be  no  reason  for  your  application  to

modify this register.

2.6.1.9  Reset Register - (4C)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Master Reset

|   |   |   |   |   |   +-------- DAC Enable

|   |   |   |   |   +------------ GF1 Master IRQ Enable

|   |   |   |   +---------------- Unused

|   |   |   +-------------------- Unused

|   |   +------------------------ Unused

|   +---------------------------- Unused

+-------------------------------- Unused

Bit 0 - GF1 Master Reset. 0 = reset, 1 = run. As long

as this is a 0, it will be held in a reset state.

Bit 1 - Enable DAC output. DAC's will not run unless this

bit is set.

Bit 2 - Master IRQ enable. This bit MUST be set to get ANY of

the GF1-generated IRQs. (wavetable,volume etc)

This register will normally contain a 0x07 when your application

is running.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 19

Chapter 2 - Hardware Information

2.6.2  Voice Specific Registers

These are the voice-specific registers.  Each voice has  its

own  bank  of  read  and  write  registers  that  alter  its

behavior.  The write registers range from 0  to  F  and  the

corresponding read  registers  range  from  80  to  8F.   To

convert from the write to the read, just add 80 hex.

Write    Read     Width   Description

0       80        8     Voice Control

1       81       16     Frequency Control

2       82       16     Starting Address (HIGH)

3       83       16     Starting Address (LOW)

4       84       16     End Address (HIGH)

5       85       16     End Address (LOW)

6       86        8     Volume Ramp Rate

7       87        8     Volume Ramp Start

8       88        8     Volume Ramp End

9       89       16     Current Volume

A       8A       16     Current Address (HIGH)

B       8B       16     Current Address (LOW)

C       8C        8     Pan Position

D       8D        8     Volume Control

E       8E        8     Active Voices (Voice independent)

-       8F        8     IRQ Status (Voice independent)

There are  several  'self-modifying'  bits  defined  in  the

following registers.  This means that  the  GF1  may  change

them at anytime on its own.  The software  must  accommodate

this phenomena.  Because of this, it's possible that the GF1

may change something immediately after your application  has

set/reset one of  the  bits.   This  is  due  to  the  GF1's

pipeline  processor  type  of  architecture.   It   does   a

read-modify-write cycle, and if  your  application  modifies

one of these bits AFTER its done the read portion and BEFORE

it does the write portion, it's possible  for  the  chip  to

perform incorrectly.  To overcome this, you  need  to  do  a

double write (with a delay in between) when those particular

bits are involved.  This delay must be at least 3 times  the

length  of  time  necessary  to  process  a  voice.   (3*1.6

microsecs).  In the lowlevel source code, this is done  with

a function called gf1_delay().  The self-modifying bits  are

designated with an (*) after the particular bit definition.

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 20

Chapter 2 - Hardware Information

2.6.2.1  Voice Control Register - (0,80)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Voice Stopped

|   |   |   |   |   |   +-------- Stop Voice

|   |   |   |   |   +------------ 16 bit data

|   |   |   |   +---------------- Loop enable

|   |   |   +-------------------- Bi-directional loop enable

|   |   +------------------------ Wave table IRQ

|   +---------------------------- Direction of movement

+-------------------------------- IRQ pending

(*)Bit 0 - 1 = Voice is stopped. This gets set by hitting the end

address (not looping) or by setting bit 1 in this reg.

Bit 1 - 1 = Stop Voice. Manually force voice to stop.

Bit 2 - 1 = 16 bit wave data, 0 = 8 bit data

Bit 3 - 1 = Loop to begin address when it hits the end address.

Bit 4 - 1 = Bi-directional looping enabled

Bit 5 - 1 = Enable wavetable IRQ. Generate an irq when the voice

hits the end address. Will generate irq even if looping

is enabled.

(*)Bit 6 - 1 = Decreasing addresses, 0 = increasing addresses. It is

self-modifying because it might shift directions when

it hits one of the loop boundaries and looping is enabled.

(*)Bit 7 - 1 = Wavetable IRQ pending. If IRQ's are enabled and

looping is NOT enabled, an IRQ will be constantly

generated until voice is stopped. This means that

you may get more than 1 IRQ if it isn't handled

properly.

2.6.2.2  Frequency Control Register - (1,81)

Bits 15-10 - Integer Portion

Bits 9-1   - Fractional Portion

Bit 0      - Not used.

This register determines the amount added to (or  subtracted

from) the current position of the voice to  determine  where

the next position will be.  This  is  how  the  interpolated

data points are determined.  If the FC register is less than

0, the GF1 will interpolate the data point  in  between  the

two actual data points.  Note that the  FC  can  be  greater

than 1.  This allows for  skipping  over  data  bytes.   The

actual frequency that it will play back is directly  related

u                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 21

Chapter 2 - Hardware Information

to the number of active voice specified (reg 8E).

2.6.2.3  Starting location HIGH - (2,82)

Bits 12-0 are the HIGH 13 bits of the address of the starting

location of the waveform. (Addr lines 19-7)

Bits 15-13 are not used.

2.6.2.4  Starting location LOW - (3,83)

Bits 15-9 are the low 7 bits of the address of the starting

location of the waveform. (Addr lines 6-0).

Bits 8-5 are the fractional part of the starting address.

Bits 4-0 are not used.

2.6.2.5  End Address HIGH - (4,84)

Bits 12-0 are the high 13 bits of the address of the ending

location of the waveform. (Addr lines 19-7)

Bits 15-13 are not used.

2.6.2.6  End Address LOW - (5,85)

Bits 15-9 are the low 7 bits of the address of the ending

location of the waveform. (Addr lines 6-0).

Bits 8-5 are the fractional part of the ending address.

Bits 4-0 are not used.

2.6.2.7  Volume Ramp Rate - (6,86)

Bits 5-0 is the amount added to (or subtracted from) the

current volume to get the next volume. The range is

from 1 t 63. The larger the number, the greater the

volume step.

Bits 7-6 defines the rate at which the increment is applied.

See the section on the volume ramping for a more

complete explanation of how this register works.

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 22

Chapter 2 - Hardware Information

2.6.2.8  Volume Ramp Start - (7,87)

Bits 7-4 Exponent

Bits 3-0 Mantissa

This register specifies the starting position of a volume ramp.

See the special section on volume ramping for a more complete

explanation of how this register works.

2.6.2.9  Volume Ramp End - (8,88)

Bits 7-4 Exponent

Bits 3-0 Mantissa

This register specifies the ending position of a volume ramp.

See the special section on volume ramping for a more complete

explanation of how this register works.

Note: The starting volume must always be less than the ending

volume. If you want the volume to ramp down, turn on the

decreasing volume bit in the Volume Control Register.

2.6.2.10  Current Volume - (9,89)

(*) Bits 15-12 Exponent

(*) Bits 11-4  Mantissa

Bits 3-0   Unused

Note: This register has 4 extra bits of precision that is

necessary for finer granularity of volume placement.

The extra bits are used during a volume ramp.

Note: This is a self-modifying value. The GF1 will update

this register as it ramps.

Note: You should always set this register equal to the value

of the beginning of the volume ramp (start OR end)

2.6.2.11  Current Location HIGH - (A,8A)

Bits 15-13  Unused

Bits 12-0   High 13 bits of address (address lines 19-7)

2.6.2.12  Current Location LOW - (B,8B)

Bits 15-9  Low 7 bits of address. (address lines 6-0)

Bits 8-0   9 bit fractional position.

L                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 23

Chapter 2 - Hardware Information

2.6.2.13  Pan Position - (C,8C)

Bits 8-4   Unused

Bits 3-0   Pan position. (0=full left, 15=full right)

2.6.2.14  Volume Ramp Control Register - (D,8D)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Ramp Stopped

|   |   |   |   |   |   +-------- Stop Ramp

|   |   |   |   |   +------------ Rollover condition

|   |   |   |   +---------------- Loop Enable

|   |   |   +-------------------- Bi-directional loop enable

|   |   +------------------------ Volume ramp IRQ enable

|   +---------------------------- Direction

+-------------------------------- IRQ pending

(*)Bit 0 - Show the ramp has stopped

Bit 1 - Manually stop the ramp.

Bit 2 - Roll over condition. This bit pertains more towards the

location of the voice rather than its volume. Its purpose

is to generate an IRQ and NOT stop (or loop). It will

generate an IRQ and the voice's address will continue

to move thru DRAM in the same direction. This can be a

very powerful feature. It allows the application to get

an interrupt without having the sound stop. This can be

easily used to implement a ping-pong buffer algorithm so

an application can keep feeding it data and there will be

no pops. Even if looping is enabled, it will not loop.

Bit 3 - Enable looping. Loop from end to start (or start to end).

Bit 4 - Enable bi-directional looping. When it hits end (or start)

it will change directions and proceed toward the other limit.

Bit 5 - Enable getting an IRQ when ramp hits end.

(*)Bit 6 - Ramp direction. 0=increasing, 1=decreasing.

(*)Bit 7 - (READ) Volume ramp IRQ pending.

2.6.2.15  Active Voices - (E,8E)

Bits 7-6  Must be set to a 1

Bits 5-0  # of voices to enable - 1.

The range is from 14 - 32. Any value less than 14 will be forced

to 14.

L                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 24

Chapter 2 - Hardware Information

2.6.2.16  IRQ Source Register - (F,8F)

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +----\

|   |   |   |   |   |   +-------- \

|   |   |   |   |   +------------  - Interrupting voice #

|   |   |   |   +---------------- /

|   |   |   +--------------------/

|   |   +------------------------ 1

|   +---------------------------- Volume Ramp IRQ pending

+-------------------------------- WaveTable IRQ pending

Bit 4-1 - Voice # (0-31) of interrupting voice

Bit 5 - ALWAYS a 1

Bit 6 - 0 = Volume Ramp IRQ occurred

Bit 7 - 0 = Wavetable IRQ occurred

Note: This is a global read only register. There is only 1 for

ALL voices. You MUST service any indicated IRQ's since a

read of this port will clear the associated IRQ bits in

the particular voice's control and/or volume control registers.

Note: It is possible that multiple voices could interrupt at

virtually the same time. In this case, this register will

behave like a fifo. When in your IRQ handler, keep reading

(and servicing) this register until you do a read with

both IRQ bits set to a 1. This means there are no voice

IRQs left to deal with.

Note: Since it is possible to get ANOTHER irq from the same voice

for the SAME reason, you must ignore any subsequent irq from

that voice while in the IRQ handler. For example, when a

voice hits its end position and generates an IRQ back to

your application, it will continue to generate IRQ's until

either the voice is stopped, the IRQ enable is turned off, or

the end location is moved.

t                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 25

Chapter 2 - Hardware Information

2.7  Global Data Low - 3X4

This register can be used to do either a 16 bit transfer for

one of the 16 bit wide GF1 registers (Start addr  high  etc)

when using a 16 bit I/O instruction or the low part of a  16

bit wide register when using an 8 bit I/O instruction.

2.8  Global Data High - 3X5

This register is used to do either an 8 bit transfer for one

of the GF1 8 bit registers or to do the high part  of  a  16

bit wide register.

h                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 26

Chapter 2 - Hardware Information

2.9  IRQ Status - 2X6

CAUTION: Note that this is at 2X6 *** NOT 3X6 ***.

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- MIDI Transmit IRQ

|   |   |   |   |   |   +-------- MIDI Receive IRQ

|   |   |   |   |   +------------ Timer 1 IRQ

|   |   |   |   +---------------- Timer 2 IRQ

|   |   |   +-------------------- Unused

|   |   +------------------------ WaveTable IRQ (any voice)

|   +---------------------------- Volume Ramp IRQ (any voice)

+-------------------------------- DMA TC IRQ (DRAM or Sample)

S                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 27

Chapter 2 - Hardware Information

2.10  Timer Control Register - 2X8

This register maps to the same location as the ADLIB board's

control register.  Writing a 4 here selects the timer stuff.

Bit 6 will be set if timer #1 has expired.  Bit  5  will  be

set it timer #2 has expired.  See timer.c for an example  of

programming the timers.

2.11  Timer Data Register - 2X9

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Timer 1 Start

|   |   |   |   |   |   +-------- Timer 2 Start

|   |   |   |   |   +------------ Unused

|   |   |   |   +---------------- Unused

|   |   |   +-------------------- Unused

|   |   +------------------------ Mask Timer 2

|   +---------------------------- Mask Timer 2

+-------------------------------- Reset Timer IRQ

Bit 0 - Start up timer #1

Bit 1 - Start up timer #1

Bit 5 - Mask off timer 2

Bit 6 - Mask off timer 1

Bit 7 - Clear Timer IRQ

x                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 28

Chapter 2 - Hardware Information

2.12  DRAM I/O - 3X7

This register is used to read or write data at the  location

pointed at by registers 43 and 44.  This is used to peek and

poke directly to DRAM.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 29

Chapter 2 - Hardware Information

2.13  Mix Control Register - 2X0

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- 0=Enable Line IN

|   |   |   |   |   |   +-------- 0=Enable Line OUT

|   |   |   |   |   +------------ 1=Enable MIC IN

|   |   |   |   +---------------- Enable latches

|   |   |   +-------------------- Combine chan1 IRQ with Chan2

|   |   +------------------------ Enable midi loopback TxD to RxD

|   +---------------------------- Control Reg Select

+-------------------------------- Unused

Bit 0 - Enable Ultrasound line Input

Bit 1 - Enable Ultrasound Line output

Bit 2 - Enable Stereo Mic Input

Bit 3 - Enable latches.This provides power to the DMA/IRQ

latches. Once these are enabled, NEVER disable them.

Disabling them will cause random IRQ's in the PC since

the DMA and IRQ lines are not being driven any more.

Bit 4 - Combine Channel 1 (GF1) irq's with Channel 2 (midi)

Bit 5 - Enable midi loopback. Any data sent out Transmit port

will be looped back into the input port.

Bit 6 - Control Register Select. When this is set to a 1, the next

IO write to 2XB will be to the IRQ control latches. When

this is set to a 0, the next IO write to 2XB will be to the

DMA channel latches. The write to 2XB for either of these

MUST occur as the NEXT IOW or else the write to 2XB will

be locked out and not occur. This is to prevent an

application that is probing for cards to accidentaly

corrupt the latches.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 30

Chapter 2 - Hardware Information

2.14  IRQ Control Register - 2XB

IRQ  control register         I/O       W         2XB

(2X0- bit 6 = 1)

Bits 2-0 Channel 1 (GF1) IRQ Selector

0=No Interrupt

1=IRQ2

2=IRQ5

3=IRQ3

4=IRQ7

5=IRQ11

6=IRQ12

7=IRQ15

Bits 5-3 Channel 2 (MIDI) IRQ selector

0=No Interrupt

1=IRQ2

2=IRQ5

3=IRQ3

4=IRQ7

5=IRQ11

6=IRQ12

7=IRQ15

Bit 6 1 = Combine Both IRQS using Channel 1's IRQ

Bit 7 Unused

Note: If the channels are sharing an IRQ, channel 2's irq

must be set to 0 and turn on bit 6. A bus conflict

will occur if both latches are programmed with the

same IRQ #.

DMA control register          I/O       W         2XB

(2X0- bit 6 = 0)

Bits 2-0 DMA Select Register 1

0=NO DMA

1=DMA1

2=DMA3

3=DMA5

4=DMA6

5=DMA7

Bits 5-3 DMA Select Register 2

0=NO DMA

1=DMA1

2=DMA3

3=DMA5

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 31

Chapter 2 - Hardware Information

4=DMA6

5=DMA7

Bit 6 - Combine Both on the same DMA channel.

Bit 7 - Unused.

Note: If the channels are sharing an DMA, channel 2's DMA

must be set to 0 and turn on bit 6. A bus conflict

will occur if both latches are programmed with the

same DMA #.

Please refer to UltraSetInterface() in init.c of the lowlevel source

code for the proper sequence for programming these latches. If the

order is not right, unpredictable things may happen.

Changing the IRQ settings will usually cause an IRQ on the OLD

IRQ because it is no longer being driven low by the latches and

it will tend to float up. That low to high transition causes and

IRQ on the PC. Normally, this is not a problem, but it is something

to be aware of.

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 32

Chapter 3 - Software Information

3.1  Lowlevel Introduction

This low level toolkit will allow developers to  write  code

at a much more basic level  than  the  other  toolkit.   Its

purpose is to allow the  programmer  direct  access  to  the

functionality of the board.  Also the code has  been  highly

optimized to make your resulting code as small and  fast  as

possible.   Currently,  9  different   memory   models   are

supplied.  They are: large, medium small and tiny  for  both

Borland  and  Microsoft  and  flat  model  with  the  Watcom

compiler.  There are also 2 different  levels  of  libraries

being shipped.  Level 0 contains the lowest  level  function

calls that talk directly  to  the  hardware.   The  Level  1

library is a little higher and contains functions that  call

level 0 functions.  The 3D libraries contain  the  functions

necessary  for  implementing  the  Focal  Point  3D  sounds.

Borland C++ 2.0, Microsoft 6.0 and Watcom C9.0/386 were used

respectively.

3.2  Library naming conventions

ultra0lb.lib Level 0, large model, Borland C

ultra1lb.lib Level 1, large model, Borland C

ult3d_lb.lib 3D, large model, Borland C

ultra0mb.lib Level 0, medium model, Borland C

ultra1mb.lib Level 1, medium model, Borland C

ult3d_mb.lib 3D, medium model, Borland C

ultra0sb.lib Level 0, small model, Borland C

ultra1sb.lib Level 1, small model, Borland C

ult3d_sb.lib 3D, small model, Borland C

ultra0tb.lib Level 0, tiny model, Borland C

ultra1tb.lib Level 1, tiny model, Borland C

ult3d_tb.lib 3D, tiny model, Borland C

ultra0lm.lib Level 0, large model, Microsoft C

ultra1lm.lib Level 1, large model, Microsoft C

ult3d_lm.lib 3D, large model, Microsoft C

ultra0mm.lib Level 0, medium model, Microsoft C

ultra1mm.lib Level 1, medium model, Microsoft C

ult3d_mm.lib 3D, medium model, Microsoft C

ultra0sm.lib Level 0, small model, Microsoft C

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 33

Chapter 3 - Software Information

ultra1sm.lib Level 1, small model, Microsoft C

ult3d_sm.lib 3D, small model, Microsoft C

ultra0tm.lib Level 0, tiny model, Microsoft C

ultra1tm.lib Level 1, tiny model, Microsoft C

ult3d_tm.lib 3D, tiny model, Microsoft C

ultra0wc.lib Level 0, Flat model, Watcom C

ultra1wc.lib Level 1, Flat model, Watcom C

ult3d_wc.lib 3D, Flat model, Watcom C

Several example applications are  supplied  on  the  toolkit

disks to show you how to interface to the libraries.  Please

look them over carefully.  They are the best way  to  get  a

handle on the way the card operates.

The next few sections will attempt to give you  an  overview

of the features the UltraSound has and how to take advantage

of them.

3.3  Main Features

32 voices

8 bit and 16 bit playback (mono and stereo)

Signed or Unsigned data

High speed DMA to/from DRAM

8 bit DMA record (mono and stereo)

1 Meg on-board DRAM

Digital volume control

Hardware volume enveloping

Line level input

Microphone input (with auto gain)

Line level output

Amplified output

Speed compensating joystick

3.4  Caveats

There are several things that a low level  programmer  needs

to be aware of to successfully program the UltraSound.   The

low  level  code  protects  against  ALL  of  these   things

happening, but it is possible for  a  programmer  to  bypass

these protections.

The DMA can only begin on a 16 or 32 byte  boundary.   An  8

bit DMA channel (0-3) must start on a 16 byte boundary while

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 34

Chapter 3 - Software Information

a 16 bit DMA channel (4-7) must start on a 32 byte boundary.

If an improper address is  supplied,  the  address  will  be

truncated down when the DMA  occurs.   The  lowlevel  memory

allocation  routines  will  ONLY  return  32  byte   aligned

addresses, so if your application uses them, this  will  not

be a problem.  If  you  chose  not  to  use  the  allocation

routines provided, be sure and follow these rules.(Note: The

3D routines require that the memory allocation  routines  be

used).

A 16 bit sample cannot be played across a 256K boundary.  An

8 bit sample can, but a 16 bit sample  cannot.   This  means

that all 16 bit samples must be less that 256K.  To  enforce

this restriction, the memory allocation routines split  each

256K bank  into  its  own  DRAM  pool.   No  memory  can  be

allocated that will cross the boundary.  Again, if you chose

not to use the memory allocation routines provided, you need

to be aware of this.

A DMA to or from the card  cannot  cross  a  256K  boundary.

Since the lowlevel code doesn't allow you to  allocate  DRAM

across a 256k boundary either, this is not usually a problem

(if you use the allocation routines).  The functions that do

DMA   to   and   from   the    card    (UltraUpload()    and

UltraDownload()), will allow you to try  and  DMA  across  a

256K boundary, but will split it up to 2 transfers.

Allowing to let the volume ramps to go to  the  rails  (0  -

4095) can cause a random oscillation of the volume  when  it

reaches the limits.  This is caused by an overshoot past the

limit due to a large step size.  The low level code protects

against this by limiting how close to the rails you can get.

3.5  Level 0 Interface Functions

3.5.1  UltraCalcRate

_____________________________________________________________

Function:  Calculate a rate for a volume ramp.

Syntax:    unsigned char UltraCalcRate(start,end,mil_secs);

unsigned int start;

unsigned int end;

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 35

UltraCalcRate

unsigned long mil_secs;

Prototype: gf1proto.h

Remarks:   This function calculates the rate necessary to

ramp the volume from the start volume to the  end

volume in a  desired  #  of  milliseconds.   This

value should be passed to UltraRampVolume.   This

is only an approximation.  The  longer  the  time

span, the less precise the result  is  likely  to

be.

Returns:   The value to pass to UltraRampVolume

See also:  UltraRampVolume,UltraRampLinearVolume

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 36

UltraClose

3.5.2  UltraClose

_____________________________________________________________

Function:  Close out the UltraSound card

Syntax:    int UltraClose(void);

Prototype: gf1proto.h

Remarks:   This function should be called before your

application exits.  It shuts down all  audio  and

puts the card in a stable state.   It  also  puts

the PC back to the state prior  to  running  your

application.  (reset irq vectors etc).

Returns:   ULTRA_OK  No problem

See also:  UltraOpen, UltraReset

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 37

UltraDownload

3.5.3  UltraDownload

_____________________________________________________________

Function:  Download a chunk of data into UltraSound DRAM.

Syntax:    int UltraDownload(dataptr,control,dram_loc,len,wait)

void *dataptr;                /* ptr to buffer in pc RAM */

unsigned char control;        /* control bits */

unsigned long dram_loc;       /* location in UltraSound */

unsigned int len;             /* # of bytes to xfer.  */

int wait;                     /* TRUE=wait for completion */

Prototype: gf1proto.h

Remarks:   This function will send a chunk of data down to

the UltraSound' DRAM.  It will transfer  'len'  #

of bytes from data_ptr (in PC)  to  dram_loc  (in

UltraSound).  If 'wait' is  TRUE,  then  it  will

wait until the transfer is complete.   If  'wait'

is FALSE, it will return as soon as  transfer  is

started.  In most cases, waiting is  usually  OK,

but if you need to get output  quickly,  you  can

start the download and then immediately  start  a

voice playing the data.  The DMA transfer is MUCH

faster than the voice playback,  so  it  will  be

able to download data ahead of the playback.  For

obvious reasons, this will not work if  you  want

to play the data backwards ...   See  Appendix  D

for a  definition  of  the  control  bits.   They

specify the type of data being sent down.

Returns:   ULTRA_OK  No problem

DMA_BUSY  Dma channel busy.  It may be busy doing a

download or upload or a record (if play and

record channels are the same).

See also:  UltraUpload,UltraDramDmaWait

i                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 38

UltraDramDmaBusy

3.5.4  UltraDramDmaBusy

_____________________________________________________________

Function:  Test to see if the DMA channel is busy

Syntax:    int UltraDramDmaBusy()

Prototype: gf1proto.h

Remarks:   This function will check to see if the DMA

to/from DRAM channel is busy.  It might be useful

so your application  doesn't  hang  around  while

waiting for a DMA transfer to complete.

Returns:   Non-zero  - Channel is still busy

0         - Channel is free

See also:  UltraWaitDramDma,UltraDownload,UltraUpload

g                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 39

UltraGoRecord

3.5.5  UltraGoRecord

_____________________________________________________________

Function:  Start up a preset record

Syntax:    int UltraGoRecord(control);

unsigned char control;

Prototype: gf1proto.h

Remarks:   This function will start up a pre-primed record.

UltraPrimeRecord can be used to do this.  It  can

also be used to restart a  indefinite  (autoinit)

record from the record handler callback.

Returns:   ULTRA_OK  No error

DMA_BUSY  This channel is busy

See

also:      UltraRecordData,UltraPrimeRecord,UltraRecordHandler

f                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 40

UltraGoVoice

3.5.6  UltraGoVoice

_____________________________________________________________

Function:  Start a voice that has been primed

Syntax:    void UltraGoVoice(voice,mode)

int voice;

unsigned char mode;

Prototype: gf1proto.h

Remarks:   This function will start up a voice that has

already  been  primed  with   setup   values   by

UltraPrimeVoice.  This can be useful if you  need

to start multiple voices  as  close  together  as

possible.   See  Appendix  C  for  the  mode  bit

definitions.

Returns:   None

See also:  UltraPrimeVoice,UltraStartVoice

3.5.7  Input/Output Source Functions

3.5.7.1  UltraDisableLineIn

_____________________________________________________________

Function:  Disable line level input.

Syntax:    void UltraDisableLineIn();

Prototype: gf1proto.h

Remarks:   If line level input is enabled and output is

enabled, the input  is  routed  directly  to  the

output and audio will be heard.  If this  is  not

desired, use this to disable line in.

Returns:   None

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 41

Input/Output Source Functions

3.5.7.2  UltraDisableMicIn

_____________________________________________________________

Function:  Disable microphone input

Syntax:    void UltraDisableMicIn();

Prototype: gf1proto.h

Remarks:   If microphone input is enabled and output is

enabled, the input  is  routed  directly  to  the

output and audio will be heard.  If this  is  not

desired, use this to disable microphone in.

Returns:   None

3.5.7.3  UltraDisableOutput

_____________________________________________________________

Function:  Turn off all output from UltraSound

Syntax:    void UltraDisableOutput();

Prototype: gf1proto.h

Remarks:   This function will disable all output from the

UltraSound.  This can be used during recording so

that the input will not be  looped  back  to  the

output.  It is  also  useful  to  disable  output

during resets etc.  It will help eliminate 'pops'

during initialization.

Returns:   None

c                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 42

Input/Output Source Functions

3.5.7.4  UltraEnableLineIn

_____________________________________________________________

Function:  Enable line level input

Syntax:    void UltraEnableLineIn();

Prototype: gf1proto.h

Remarks:   Turn on the line level input.  If you are not

recording from the line input, it is probably not

desirable to have this enabled since it  will  be

looped back to the output (if output is enabled).

Returns:   None

3.5.7.5  UltraEnableMicIn

_____________________________________________________________

Function:  Turn on the microphone input

Syntax:    void UltraEnableMicIn();

Prototype: gf1proto.h

Remarks:   This function should be called when you want to

record from the microphone.  It  is  possible  to

have both the microphone input enabled  and  line

level input enabled.  If you  are  not  recording

from the microphone, it is recommended that it be

disabled.  This will reduce noise on the output.

Returns:   None

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 43

Input/Output Source Functions

3.5.7.6  UltraEnableOutput

_____________________________________________________________

Function:  Enable any output from UltraSound

Syntax:    void UltraEnableOutput();

Prototype: gf1proto.h

Remarks:   This function must be called to enable any output

from the UltraSound.  If output is  not  enabled,

you will get no audio output.  It can be used  to

mute output at a particular  time.   (Like  reset

..).

Returns:   None

3.5.7.7  UltraGetLineIn

_____________________________________________________________

Function:  Return the current state of line level input

Syntax:    int UltraGetLineIn();

Prototype: gf1proto.h

Remarks:   This can be useful if you want to change the

state of the line level input and then restore it

back to the original state.

Returns:   Current line level input state (ON or OFF)

3.5.7.8  UltraGetOutput

_____________________________________________________________

Function:  Return the current output enable state.

Syntax:    int UltraGetOutput();

Prototype: gf1proto.h

Remarks:   This can be useful if you want to change the

state of the output and then restore it  back  to

the original state.

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 44

Input/Output Source Functions

Returns:   None

3.5.7.9  UltraGetMicIn

_____________________________________________________________

Function:  Return the current state of the microphone input.

Syntax:    int UltraGetMicIn();

Prototype: gf1proto.h

Remarks:   This can be useful if you want to change the

state of the microphone input and then restore it

back to the original state.

Returns:   None

h                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 45

Interrupt Handling

3.5.8  Interrupt Handling Functions

The following 8 functions can be used to  define  a  handler

for events that happen under interrupt  on  the  UltraSound.

These are NOT interrupt handlers but are callbacks from  the

interrupt handler.  This  means  that  they  should  not  be

defined as interrupt type functions but must adhere  to  the

general rules of interrupt code.  No  DOS  calls  should  be

made and care must be taken not to cause problems with  code

running in the foreground.  All these functions  return  the

old callback address so chaining could be done  if  desired.

This not usually necessary.  It is  also  not  necessary  to

restore the handler back to the old one  when  exiting  your

application.   However,  UltraClose()  MUST  be  called   to

restore the actual interrupt handler.  These  will  only  be

called if the  interrupt  for  the  particular  function  is

enabled  in  the  mode  parameter.   (See  the   appropriate

Appendix for the bit definitions.) It is also not  necessary

to set up a callback since it has a default associated  with

it, but then why bother enabling the interrupt if you  don't

want to know when it finishes?

You should be able to make any level0 or  level1  UltraSound

library calls from within the interrupt  handler.   All  the

library functions protect themselves from being  interrupted

while doing critical operations.

3.5.8.1  UltraDramTcHandler

_____________________________________________________________

Function:  Define a callback for DMA complete

Syntax:    PFV *(UltraDramTcHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function will be called when a DMA transfer

to/from the UltraSound DRAM  is  complete.   This

can be called to start up another transfer.

Nothing is passed to this handler.

Returns:   Old handler

m                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 46

Interrupt Handling

3.5.8.2  UltraMidiXmitHandler

_____________________________________________________________

Function:  Define a callback for MIDI xmit IRQ

Syntax:    PFV *(UltraMidiXmitHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function will be called on a MIDI transmit

empty IRQ.  This can be used  to  send  out  MIDI

data under interrupt.

The midi status byte is passed  to  the  handler.

This is used to determine if there were an errors

in transmission.  The status bits are defined  in

Chapter 2.

Returns:   Old handler

3.5.8.3  UltraMidiRecvHandler

_____________________________________________________________

Function:  Define a callback for a MIDI recv IRQ

Syntax:    PFV *(UltraMidiRecvHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function will be called when a character

is received in the MIDI input port.  This can  be

used  to  get  data  from  the  MIDI  port  under

interrupt.  The midi port status  and  midi  data

are passed to your handler.  The status bits  are

defined in Appendix D.  Your  handler  should  be

defined like this:

PFV handler(status,data);

unsigned char status;

unsigned char data;

Returns:   Old handler

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 47

Interrupt Handling

3.5.8.4  UltraTimer1Handler

_____________________________________________________________

Function:  Define a callback from Timer #1

Syntax:    PFV *(UltraTimer1Handler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   There are 2 general purpose timers available

on  the  UltraSound.    They   are   functionally

identical  to  the  Adlib  timers.   Nothing   is

currently   passed   to   this   handler.     See

UltraTimer2Handler().

Returns:   Old handler

3.5.8.5  UltraTimer2Handler

_____________________________________________________________

Function:  Define a callback for Timer #2

Syntax:    PFV *(UltraTimer2Handler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   There are 2 general purpose timers available

on  the  UltraSound.    They   are   functionally

identical  to  the  Adlib  timers.   Nothing   is

currently   passed   to   this   handler.     See

UltraTimer1Handler().

Returns:   Old handler

s                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 48

Interrupt Handling

3.5.8.6  UltraWaveHandler

_____________________________________________________________

Function:  Define a callback for wavetable IRQ's

Syntax:    PFV *(UltraWaveHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function will be called when a voice

generates a wavetable IRQ.  This happens  when  a

voice hits the end of its wave and interrupts are

enabled.  It will happen even if  looping  is  on

(i.e.  the voice keeps  playing).   Normally,  it

would be used to signify that  a  voice  is  done

playing.  This IRQ can  be  useful  for  starting

another voice or counting the # of times  that  a

voice goes thru a loop before it is stopped.  The

voice # that generated the IRQ is passed back  to

your handler.

Returns:   Old handler

3.5.8.7  UltraVolumeHandler

_____________________________________________________________

Function:  Define a callback for volume ramp complete

Syntax:    PFV *(UltraVolumeHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function can be used to generate a volume

envelope (attack,decay,sustain,release)  This  is

done by changing to the appropriate volume  ramps

in the handler to handle the  next  part  of  the

envelope.  The voice # causing the  irq  will  be

passed back to your handler.

Returns:   Old handler

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 49

Interrupt Handling

3.5.8.8  UltraRecordHandler

_____________________________________________________________

Function:  Define a callback for a DMA record complete

Syntax:    PFV *(UltraUltraRecordHandler(handler));

PFV *handler;

Prototype: gf1proto.h

Remarks:   This function is called when a buffer that was

being recorded is full.  Normally, this would  be

used to let  the  application  start  up  another

record.  A double buffering scheme could be  used

to record data continuously.

Nothing is passed to this handler.

As long as the DMA  channels  for  recording  and

playback are different, the UltraSound is capable

of simultaneously  recording  and  playback.   At

high data rates, throughput may be a problem that

your application will have to deal with.

Returns:   Old handler

t                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 50

UltraMaxAlloc

3.5.9  UltraMaxAlloc

_____________________________________________________________

Function:  Find the size of the  largest  allocatable  block

left

Syntax:    unsigned long UltraMaxAlloc();

Prototype: gf1proto.h

Remarks:   This function will return the largest block

of DRAM (in bytes) that can be  allocated.   This

can be useful for determining whether  or  not  a

patch can be loaded.  The maximum size of a block

is 256K.

Returns:   # of bytes in largest available block.

See also:  UltraMemAlloc,UltraMemFree

m                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 51

UltraMemAlloc

3.5.10  UltraMemAlloc

_____________________________________________________________

Function:  Allocate a chunk of UltraSound DRAM

Syntax:    int UltraMemAlloc(size, location);

unsigned long size;

unsigned long *location;

Prototype: gf1proto.h

Remarks:   This function allocates a chunk of DRAM of

'size' bytes from  the  UltraSound.   The  memory

allocation structures are set up in  UltraOpen().

A first-fit algorithm is used so it is up to  the

programmer   to    avoid    too    much    memory

fragmentation.  'Location' is filled in with  the

DRAM location  of  the  start  of  the  chunk  of

memory.   The  memory  returned  will  ALWAYS  be

aligned on a 32 byte boundary so  that  the  DRAM

can be DMA'ed  into.   Also,  the  size  will  be

rounded UP to the next 32 byte boundary.

Returns:   ULTRA_OK             No Error

NO_MEMORY            No chunk of DRAM large enough

See also:  UltraMemInit,UltraMemFree

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 52

UltraMemFree

3.5.11  UltraMemFree

_____________________________________________________________

Function:  Free a chunk of UltraSound DRAM

Syntax:    int UltraMemFree(size,location)

unsigned long size;

unsigned long location;

Prototype: gf1proto.h

Remarks:   Return a chunk of DRAM back to the memory pool.

Be sure that the proper size chunk is returned so

the memory structures  are  not  corrupted.   The

size will automatically be rounded UP to the next

32 byte boundary.

Returns:   ULTRA_OK             No problem

CORRUPT_MEM          Memory structs have been corrupted

See also:  UltraMemInit, UltraMemAlloc

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 53

UltraMemInit

3.5.12  UltraMemInit

_____________________________________________________________

Function:  Initialize memory free pool structures.

Syntax:    unsigned long UltraMemInit();

Prototype: gf1proto.h

Remarks:   This function sets up the UltraSound DRAM so that

UltraMemAlloc() and UltraMemFree() will work.  It

is called in UltraOpen().  It can  be  called  at

any time if an application wants to clean up  all

its allocated or corrupted memory structures.

Returns:   Number of K of DRAM found on the UltraSound.

If an application wishes to reserve  a  chunk  of

DRAM outside  of  the  memory  pool,  a  variable

called _ultra_reserved_dram must be set  up  with

the # of bytes to reserve  BEFORE  UltraMemInit()

is called.  This reserved chunk will start at  0.

The reserved chunk must be less than 256K.

See also:  UltraMemAlloc, UltraMemFree

h                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 54

UltraMidiDisableRecv

3.5.13  UltraMidiDisableRecv

_____________________________________________________________

Function:  Disable midi receive data interrupt

Syntax:    void UltraMidiDisableRecv();

Prototype: gf1proto.h

Remarks:   This function will disable the receive

data interrupts from the midi.   This  should  be

disabled before leaving your application.

Returns:   None

See also:  UltraMidiEnableRecv, UltraMidiRecvhandler

i                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 55

UltraDisableMidiXmit

3.5.14  UltraDisableMidiXmit

_____________________________________________________________

Function:  Disable midi transmit irqs

Syntax:    void UltraDisableMidiXmit();

Prototype: gf1proto.h

Remarks:   This function will turn off midi

transmit interrupts.  It MUST be called when  you

are through sending data.

Returns:   None

See also:  UltraMidiXmitHandler, UltraMidiEnableXmit

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 56

UltraMidiEnableRecv

3.5.15  UltraMidiEnableRecv

_____________________________________________________________

Function:  Enable receive data interrupts for midi port

Syntax:    void UltraMidiEnableRecv();

Prototype: gf1proto.h

Remarks:   This function will enable receive data

interrupts from the midi  port.   It  is  usually

necessary to set up a callback function  so  your

application can process the data.

Returns:   None

See also:  UltraMidiRecvhandler, UltraMidiDisableRecv

c                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 57

UltraEnableMidiXmit

3.5.16  UltraEnableMidiXmit

_____________________________________________________________

Function:  Enable transmit interrupts from MIDI

Syntax:    void UltraEnableMidiXmit();

Prototype: gf1proto.h

Remarks:   This function will enable transmit data

interrupts  to  be  generated  as  each  byte  is

transmitted out  the  midi  port.   Note  that  a

transmit IRQ will be generated as soon as the IRQ

is  enabled  unless  a  character  is  sent   out

immediately  prior  to  enabling  it.   This   is

because  the  xmit  buffer  is  initially   empty

(unless primed) so it will pop an IRQ.  Also note

that you MUST disable this irq when you  are  not

sending any more data or else you will be hung up

getting transmit ready irqs.

Returns:   None

See also:  UltraMidiDisableXmit

s                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 58

UltraMidiRecv

3.5.17  UltraMidiRecv

_____________________________________________________________

Function:  Read a character from the midi port.

Syntax:    unsigned char UltraMidiRecv();

Prototype: gf1proto.h

Remarks:   This function is used to read a character from

the midi port.  It assumes that the character  is

waiting.  The character is there if it got to the

Midi receive interrupt callback  function  or  if

you have polled the  status  and  determined  the

receive buffer is full.

Returns:   Midi data

See also:  UltraMidiRecvHandler,UltraMidiStatus

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 59

UltraMidiReset

3.5.18  UltraMidiReset

_____________________________________________________________

Function:  Reset the MIDI port.

Syntax:    void UltraMidiReset();

Prototype: gf1proto.h

Remarks:   This function should be used to ensure

that the midi port is ready  to  use.   All  midi

IRQs will be disabled by this call.

Returns:   None

See also:  UltraEnableMidiXmit, UltraEnableMidiRecv

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 60

UltraMidiStatus

3.5.19  UltraMidiStatus

_____________________________________________________________

Function:  Read the MIDI status bits

Syntax:    unsigned char UltraMidiStatus()

Prototype: gf1proto.h

Remarks:   This function returns the current MIDI

port  status  bits.   See  Chapter  2   for   the

definition of all the bits.  This can be used  to

determine if an error has occurred or if the port

is ready to be read or written.

Returns:   The status bits.  See Chapter 2.

See also:  UltraMidiXmit,UltraMidiRecv

d                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 61

UltraMidiXmit

3.5.20  UltraMidiXmit

_____________________________________________________________

Function:  Send a byte out the MIDI port.

Syntax:    void UltraMidiXmit(data)

unsigned char data;

Prototype: gf1proto.h

Remarks:   This function will send a byte out the

MIDI  data  port.   If  irqs  are   enabled,   an

interrupt will be generated  when  the  character

has been transmitted.

Returns:   None

See also:  UltraMidiXmitHandler

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 62

UltraOpen

3.5.21  UltraOpen

_____________________________________________________________

Function:  Open and initialize the lowlevel code.

Syntax:    #include "extern.h"

int UltraOpen(config,voices);

ULTRA_CFG *config;

int voices;          /* # of active voices (14-32) */

Prototype: gf1proto.h

Remarks:   This function should ALWAYS be called to

initialize the UltraSound.  It will probe for the

card and program the irq  and  dma  latches.   It

will then disable line and microphone  input  and

enable output.  It also  initializes  the  memory

structures.   The  #  of  active  voices  is   an

important parameter to the UltraSound.  The fewer

the #  of  voices,  the  more  oversampling  that

occurs on playback.  That  will  make  the  sound

much cleaner.  However, if you need  more  voices

to make more sounds, allocate them as  necessary.

In general, only make the minimum  number  active

as possible (14 is the minimum).

Returns:   ULTRA_OK             No problem

NO_ULTRA             No UltraSound card found

BAD_NUM_OF_VOICES    # of active voices not in range

See also:  UltraClose,UltraProbe,UltraMemInit

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 63

UltraPeekData

3.5.22  UltraPeekData

_____________________________________________________________

Function:  Examine any DRAM location on UltraSound

Syntax:    unsigned char UltraPeekData(baseport,location);

unsigned int baseport;

unsigned long location;

Prototype: gf1proto.h

Remarks:   This function is used to allow an application

to look at  any  location  in  UltraSound's  DRAM

whenever  it  wishes.   This  can  be  handy  for

obtaining VU information or any other time it  is

nice to know what is in DRAM.  Be aware that  the

data will probably be in  twos  compliment  form.

The data MUST be in twos compliment form for  the

UltraSound to play it.  If your original data was

in one's compliment, it  was  probably  converted

when it was DMA'ed down to the UltraSound.

If the data that you want is 16 bit data, be sure

and peek both locations and  do  any  appropriate

conversions.   The  data  will  be  in   low/high

format.  That means that the low byte of the data

will be in the even byte and the high  byte  will

be in the odd byte.

Returns:   Data byte at location.

See also:  UltraPokeData, UltraDownload

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 64

UltraPing

3.5.23  UltraPing

_____________________________________________________________

Function:  Quick check to see if UltraSound is present

Syntax:    int UltraPing(baseport)

int baseport;

Prototype: gf1proto.h

Remarks:   This function will determine if an UltraSound is

present by attempting to read and  write  to  its

DRAM.  This function  assumes  that  at  least  a

simple reset has been done since powerup so  that

the board is no longer in a reset state.   If  it

is on a reset state, this  function  will  ALWAYS

fail.  UltraProbe() will pull a quick  reset  and

then call UltraPing().

Returns:   ULTRA_OK  Found a board.

NO_ULTRA  No board at this I/O location

See also:  UltraProbe

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 65

UltraPokeData

3.5.24  UltraPokeData

_____________________________________________________________

Function:  Poke a byte into UltraSound DRAM

Syntax:    void UltraPokeData(baseport,location,data);

unsigned int baseport;

unsigned long location;

unsigned char data;

Prototype: gf1proto.h

Remarks:   Poke an 8 bit value directly into UltraSound

DRAM.  This can be useful to set the value of the

location that a voice  is  pointing  to.   It  is

often desirable to point a voice to a known value

since its output  is  ALWAYS  summed  in  to  the

output even if the  voice  is  not  running.   Be

aware that there is no  conversion  of  the  data

that  is  being  poked  into  DRAM.   Since   the

UltraSound only plays twos compliment data,  make

sure that the data you  are  poking  is  in  that

format.  Also be careful with 16 bit data.

Returns:   None

See also:  UltraPeekData,UltraDownload

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 66

UltraPrimeRecord

3.5.25  UltraPrimeRecord

_____________________________________________________________

Function:  Prime the record DMA channel

Syntax:    int UltraPrimeRecord(pc_ptr,size,repeat);

void far *pc_ptr;

unsigned int size;

int repeat;

Prototype: gf1proto.h

Remarks:   This function will setup the DMA channel to do

a record, but do not start it.  This can be  used

to help synchronize events.  Programming the  DMA

channel can  take  enough  time  so  that  a  few

samples may be lost.  (Depending on sample rate).

This function will help alleviate this problem by

doing the programming ahead of time.

Returns:   ULTRA_OK  No problem

DMA_BUSY  Dma channel busy.  It may be busy doing a

download or upload or a record (if play and

record channels are the same).

See also:

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 67

UltraPrimeVoice

3.5.26  UltraPrimeVoice

_____________________________________________________________

Function:  This function primes a voice with the

pertinent values but does NOT start it.

Syntax:

unsigned char UltraPrimeVoice(voice,begin,start,end,mode)

int voice;           /* voice to prime */

unsigned long begin; /* begin loc in DRAM */

unsigned long start; /* start loop loc in DRAM */

unsigned long end;   /* end location in DRAM */

unsigned char mode;  /* mode to set voice to (loop etc) */

Prototype: gf1proto.h

Remarks:   This function is used to do all the setup

necessary to start a voice but does NOT start  it

up.  This can be useful if you want to start more

than   1   voice   at   the   same   time.    Use

UltraPrimeVoice() to do all the  necessary  setup

and then use UltraGoVoice() to start the  voices.

UltraStartVoice calls UltraPrimeVoice()  and  the

UltraGoVoice().

Returns:   An updated mode value.  The mode may be modified on

the basis of the location parameters.  This altered value

should be the one passed to UltraGoVoice().

See also:  UltraStartVoice,UltraGoVoice

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 68

UltraProbe

3.5.27  UltraProbe

_____________________________________________________________

Function:  Probe for the existence of UltraSound

Syntax:    int UltraProbe(base_port);

unsigned int base_port;

Prototype: gf1proto.h

Remarks:   This function probes for the existence of an

UltraSound card at the specified base  port.   An

application  could  call  this   before   calling

UltraOpen()  to  see  if  a  card   is   present.

UltraOpen()  calls  this   routine   also.    The

difference between UltraProbe()  and  UltraPing()

is that UltraProbe() will pull a  reset  to  make

sure the board is running.   UltraPing()  assumes

this has already been done.

Returns:   ULTRA_OK  No problem

NO_ULTRA  No card found at this base port

See also:  UltraOpen,UltraPing

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 69

UltraRampVolume

3.5.28  UltraRampVolume

_____________________________________________________________

Function:  Ramp a voice's volume up or down.

Syntax:    void UltraRampVolume(voice,start,end,rate,mode)

int voice;           /* voice to use */

unsigned int start;  /* start value (0-511) */

unsigned int end;    /* end value (0-511) */

unsigned char rate;  /* range/rate value to stuff */

unsigned char mode;  /* mode to run the volume (loop etc) */

Prototype: gf1proto.h

Remarks:   Start a volume ramp from a starting volume to the

ending volume.  The rate at which the ramp occurs

can be  calculated  using  UltraCalcRate().   The

mode  determines  the  looping  and  interrupting

characteristics of the ramp.  If  you  choose  to

have it interrupt at the end  of  the  ramp,  you

should set up an function to call  for  a  volume

interrupt.   This  is  done  with  the   function

UltraVolumeHandler().  The mode bits are  defined

in Appendix B.

Returns:   None

See also:  UltraCalcRate,UltraVolumeHandler

T                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 70

UltraReadRecordPosition

3.5.29  UltraReadRecordPosition

_____________________________________________________________

Function:  Return the # of bytes recorded so far.

Syntax:    unsigned int UltraReadRecordPosition();

Prototype: gf1proto.h

Remarks:   This function can be used to monitor the amount

data recorded so far.

Returns:   Number of BYTES recorded so far.

See also:  UltraRecordData

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 71

UltraReadVoice

3.5.30  UltraReadVoice

_____________________________________________________________

Function:  Read a voice's current location.

Syntax:    unsigned long UltraReadVoice(voice)

int voice;

Prototype: gf1proto.h

Remarks:   This routine returns the absolute position

of this voice in DRAM

Returns:   Current position in DRAM

See also:  UltraSetVoice

S                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 72

UltraReadVolume

3.5.31  UltraReadVolume

_____________________________________________________________

Function:  Read a voice's current volume.

Syntax:    unsigned int UltraReadVolume(voice)

int voice;

Prototype: gf1proto.h

Remarks:   This routine returns the current volume of this

voice.   This  can  be  useful   when   used   in

conjunction  with  volume   ramps.    The   value

returned is NOT in linear units.  (0-511)

Returns:   Current volume value (0-4095)

See also:  UltraSetVolume,UltraRampVolume

.                      Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 73

UltraRecordData

3.5.32  UltraRecordData

_____________________________________________________________

Function:  Record some data with the UltraSound

Syntax:    int UltraRecordData(pc_ptr,control,size,wait,repeat)

void far *pc_ptr;

unsigned char control;

unsigned int size;

int wait;

int repeat;

Prototype: gf1proto.h

Remarks:   This function will record a buffer of data from

UltraSound.  It can be in either  8-bit  mono  or

stereo.  In stereo, the left byte is  first  then

the right.  If  mono  is  being  used,  the  left

channel is the one that is sampled.  See Appendix

E for the a description of the recording  control

bits.  If wait is set to a non-zero  value,  then

this function will not return  until  the  buffer

has been filled.  If repeat is set to a  non-zero

value, then the DMA channel will  be  set  up  in

autoinit mode  so  that  the  recording  is  done

indefinitely.  If this is done, then  the  buffer

MUST reside completely in 1 64K page of  PC  RAM.

Also,  it  is  probably   necessary   that   your

application hooks to the record handler  so  that

the control register on the UltraSound can be hit

to     restart     the     recording     process.

(UltraGoRecord()) This will be very  quick  since

the PC DMA controller will not be re-programmed.

Returns:   ULTRA_OK  No error

DMA_BUSY  Sampling DMA channel is busy.

BAD_DMA_ADDR         Autoinit Buffer crosses 64K page.

See also:  UltraWaitRecordDma,UltraSetRecordFrequency

UltraGoRecord

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 74

UltraRecordDmaBusy

3.5.33  UltraRecordDmaBusy

_____________________________________________________________

Function:  See if record DMA channel is busy

Syntax:    int UltraRecordDmaBusy();

Prototype: gf1proto.h

Remarks:   This function checks to see if the record DMA

channel is busy.  It might be busy doing a record

or playback.  (If the record & playback  channels

are the same).

Returns:   ULTRA_OK  No error

DMA_BUSY  This channel is not free to use yet.

See also:  UltraRecordData

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 75

UltraReset

3.5.34  UltraReset

_____________________________________________________________

Function:  Pull a full reset on the UltraSound

Syntax:    int UltraReset(voices)

int voices;

Prototype: gf1proto.h

Remarks:   This function is called by UltraOpen() to make

sure the card is in a known state.   UltraClose()

also calls this function.

Returns:   ULTRA_OK             No problem

BAD_NUM_OF_VOICES    # of voices not in range

See also:  UltraOpen, UltraClose

.                      Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 76

UltraSetBalance

3.5.35  UltraSetBalance

_____________________________________________________________

Function:  Set a voice's pan position

Syntax:    void UltraSetBalance(voice,data)

int voice;

int data;            /* (0-15) */

Prototype: gf1proto.h

Remarks:   This function sets the voice's position somewhere

between right and left.  A 0 will place the audio

all the way to the left.  A 15 will put the sound

all the way to the right.

Returns:   None

See also:

i                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 77

UltraSetFrequency

3.5.36  UltraSetFrequency

_____________________________________________________________

Function:  Set a voice's playback frequency

Syntax:    void UltraSetFrequency(voice,speed_hz)

int voice;

unsigned long speed_hz;

Prototype: gf1proto.h

Remarks:   This function sets the voice's playback rate to

the specified rate.  The number of active  voices

is taken into account when making the appropriate

calculations.

Returns:   None

See also:

g                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 78

UltraSetLoopMode

3.5.37  UltraSetLoopMode

_____________________________________________________________

Function:  Set a voice's loop mode

Syntax:    void UltraSetLoopMode(voice,mode)

int voice;

unsigned char mode;

Prototype: gf1proto.h

Remarks:   This function will set this voice's looping

mode to the specified mode.  See Appendix  C  for

the definition of these bits.

Returns:   None

See also:

A                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 79

UltraSetRecordFrequency

3.5.38  UltraSetRecordFrequency

_____________________________________________________________

Function:  Set the recording rate.

Syntax:    void UltraSetRecordFrequency(rate)

unsigned long rate;  /* in hertz */

Prototype: gf1proto.h

Remarks:   This function sets the record rate to the

specified rate.  Since the UltraSound uses the PC

DMA channel to do the sampling directly  into  PC

RAM, no voice is specified.

Returns:   None

See also:

r                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 80

UltraSetVoice

3.5.39  UltraSetVoice

_____________________________________________________________

Function:  Set a voice to an absolute position

Syntax:    void UltraSetVoice(voice,location)

int voice;                    /* voice to set */

unsigned long location;       /* location in ultra DRAM */

Prototype: gf1proto.h

Remarks:   This function sets a voice's current position to

an absolute location.  This can be useful to  set

a voice to a location with a  known  value  since

all voices current locations are summed in to the

output even if the voice is not running.   'Pops'

in the audio may result if a voice is  set  to  a

location that is not proper.

Returns:   None

See also:

v                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 81

UltraSetVoiceEnd

3.5.40  UltraSetVoiceEnd

_____________________________________________________________

Function:  Set a voice's end position.

Syntax:    void UltraSetVoiceEnd(voice,end)

int voice;           /* voice to start */

unsigned long end;   /* end location in ultra DRAM */

Prototype: gf1proto.h

Remarks:   This function sets a new endpoint for the

specified  voice.   Used  in   conjunction   with

UltraSetLoopMode() to turn looping off, a sampled

decay can be implemented.

Returns:   None

See also:  UltraSetLoopMode

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 82

UltraSetVolume

3.5.41  UltraSetVolume

_____________________________________________________________

Function:  Set a voice's current volume.

Syntax:    void UltraSetVolume(voice,volume)

int voice;

unsigned int volume;          /* (0-4095) */

Prototype: gf1proto.h

Remarks:   This function sets the volume of the voice to a

specific value.  The range is from 0 to 4095  and

is    NOT    linear.     It    is    logarithmic.

UltraSetLinearVolume() can do that.

Returns:   None

See also:  UltraSetLinearVolume

s                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 83

UltraSizeDram

3.5.42  UltraSizeDram

_____________________________________________________________

Function:  Find the amount of DRAM on UltraSound

Syntax:    int UltraSizeDram()

Prototype: gf1proto.h

Remarks:   This function could be used by an application to

determine  how  much  it  can   load   into   the

UltraSound.    UltraMemInit()   calls   this   to

determine how much can be  used  for  its  memory

pool.

Returns:   # of K found on the UltraSound.

See also:  UltraMemInit

f                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 84

UltraStartTimer

3.5.43  UltraStartTimer

_____________________________________________________________

Function:  Start up a Timer

Syntax:    void UltraStartTimer(timer,duration)

int timer;                    /* 1 or 2 */

unsigned char duration;       /* 0 - 255 ticks */

Prototype: gf1proto.h

Remarks:   This function can be used to start  up

one of two available hardware timers.   Timer  #1

is an 80 microsecond timer.  Timer #2  is  a  320

microsecond timer.  You supply  the  #  of  ticks

before the timer stops.  When the timer stops, it

ALWAYS calls the callback  function  defined  for

it.  If you don't supply one, a default is  used.

This can be used to  trigger  various  real  time

events.   Please  remember  that   the   callback

routine is called  directly  from  the  interrupt

handler, so you must be careful what  you  do  in

the callback routine.  Note  that  the  timer  is

automatically restarted  after  it  ticks.   Your

application must explicitly call UltraStopTimer()

to shut it off.

Returns:   None

See also:  UltraStopTimer,UltraTimerStopped

UltraTimer1Handler,UltraTimer2Handler.

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 85

UltraStartVoice

3.5.44  UltraStartVoice

_____________________________________________________________

Function:  Start a voice playback.

Syntax:    void UltraStartVoice(voice,begin,start,end,mode)

int voice;           /* voice to start */

unsigned long begin; /* start loc in DRAM */

unsigned long start; /* start loop loc in DRAM */

unsigned long end;   /* end location in DRAM */

unsigned char mode;  /* mode to run voice (loop etc) */

Prototype: gf1proto.h

Remarks:   This function will start up a voice.  The voice

will begin playback at the 'begin'  and  continue

to 'end' if looping is on, then it will loop back

to 'start' (or play backwards to start) and  then

continue to 'end'.   The  method  of  looping  is

determined by 'mode'.  See  Appendix  C  for  the

definition of these bits.

NOTE: To play a sample backwards, 'begin'  should

be set to address that you want it to  start  at,

start must be a lower address than the end.   Now

the sample will  begin  playing  at  begin,  with

decreasing addresses thru the end location  until

it hits the start location.  It  will  then  loop

back to the end position, (if looping enabled)

Forward playing.

================

-------->------->-------->------|

^---------<------<-|

Begin ...... Start ............. End

location ... location  ......... location

Backward playing.

=================

|------<-----<------<------<----------

|--------->----->---->---^

Start ............... End ........... Begin

location ............ location .......location

Returns:   None

See also:  UltraSetEnd,UltraLoopMode

h                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 86

UltraStartVoice

P                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 87

UltraStopTimer

3.5.45  UltraStopTimer

_____________________________________________________________

Function:  Stop one of the 2 timers

Syntax:    void UltraStopTimer(timer)

int timer;                    /* 1 or 2 */

Prototype: gf1proto.h

Remarks:   This function shuts off a particular timer.

Returns:   None

See also:  UltraStartTimer

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 88

UltraStopVoice

3.5.46  UltraStopVoice

_____________________________________________________________

Function:  Stop a voice.

Syntax:    void UltraStopVoice(voice)

int voice;

Prototype: gf1proto.h

Remarks:   This function is used to stop a currently

playing voice.

Returns:   None

See also:  UltraStartVoice,UltraSetEnd

S                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 89

UltraStopVolume

3.5.47  UltraStopVolume

_____________________________________________________________

Function:  Stop a volume ramp.

Syntax:    void UltraStopVolume(voice)

int voice;

Prototype: gf1proto.h

Remarks:   This function is used to stop a currently

active volume ramp.

Returns:   None

See also:  UltraRampVolume, UltraRampLinearVolume

S                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 90

UltraTimerStopped

3.5.48  UltraTimerStopped

_____________________________________________________________

Function:  Check to see if a timer is not running any more.

Syntax:    int UltraTimerStopped(timer)

int timer;

Prototype: gf1proto.h

Remarks:   This function will return non-zero if the timer

is stopped.

Returns:   0 if still running.

non-zero if stopped.

See also:  UltraStartTimer,UltraStopTimer

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 91

UltraTrimJoystick

3.5.49  UltraTrimJoystick

_____________________________________________________________

Function:  Set the trim voltage on the joystick port

Syntax:    void UltraTrimJoystick(value);

unsigned char value;

Prototype: gf1proto.h

Remarks:   This function is used to set the speed

compensation value on the joystick  port  on  the

UltraSound.  The faster the computer, the smaller

this value should be.  This allows  all  software

that reads  the  joystick  to  return  consistent

joystick positions regardless of the speed of the

machine.   This  is  normally  not   needed   and

probably   should   never   be   used   in   your

application.  The utility Ultrajoy.exe is used to

set this up in your autoexec.bat.

Returns:   None

See also:

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 92

UltraUpload

3.5.50  UltraUpload

_____________________________________________________________

Function:  Dma a section of UltraSound DRAM into PC Ram

Syntax:    int UltraUpload(dataptr,control,dram_loc,len,wait)

void *dataptr;                /* ptr to buffer in pc RAM */

unsigned char control;        /* control bits */

unsigned long dram_loc;       /* location in UltraSound */

unsigned int len;             /* # of bytes to read.  */

int wait;                     /* TRUE=wait for completion */

Prototype: gf1proto.h

Remarks:   This function will retrieve a chunk of data from

the UltraSound's DRAM.  It will transfer 'len'  #

of bytes to data_ptr (in PC)  from  dram_loc  (in

UltraSound).  If 'wait' is  TRUE,  then  it  will

wait until the transfer is complete.   If  'wait'

is FALSE, it will return as soon as  transfer  is

started.  See Appendix D for a definition of  the

control bits.  They  specify  the  type  of  data

being retrieved.

Returns:   ULTRA_OK  No problem

DMA_BUSY  Dma channel busy.  It may be busy doing a

download or upload or a record (if play and

record channels are the same).

See also:  UltraDownload,UltraDramDmaWait

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 93

UltraVectorVolume

3.5.51  UltraVectorVolume

_____________________________________________________________

Function:  Ramp a volume from here to new end point

Syntax:    void UltraVectorVolume(voice,end_idx,rate,mode)

int voice;

unsigned int end;    /* end volume to go to */

unsigned char rate;

unsigned char mode;  /* mode to run the vol ramp in ...  */

Prototype: gf1proto.h

Remarks:   This function can be used to ramp from an

unknown place to a new place.  It  is  useful  if

you  are  doing  volume  envelopes  and  need  to

restart the attack/decay sequence at any time.

Returns:   None

See also:  UltraRampVolume,UltraStopVolume

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 94

UltraVersion

3.5.52  UltraVersion

_____________________________________________________________

Function:  Return the version of the low level code

Syntax:    void UltraVersion(major,minor);

int *major;

int *minor;

Prototype: gf1proto.h

Remarks:   This function can be used to track the

version of the low level code being used.

Returns:   Major and minor version of low level code.

See also:

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 95

UltraVoiceStopped

3.5.53  UltraVoiceStopped

_____________________________________________________________

Function:  Return the state of a voice.

Syntax:    int UltraVoiceStopped(voice);

int voice;

Prototype: gf1proto.h

Remarks:   This function is used to determine the current

state of the voice.  It can be used to see  if  a

wave has finished yet.

Returns:   0         Voice is running

non-zero  Voice is stopped

See also:

e                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 96

UltraVolumeStopped

3.5.54  UltraVolumeStopped

_____________________________________________________________

Function:  Determine if volume ramp is running

Syntax:    int UltraVolumeStopped(voice);

int voice;

Prototype: gf1proto.h

Remarks:   This function is used to determine the current

state of the volume of the voice.  It can be used

to see if a volume ramp has finished yet.

Returns:   0         Volume ramp is running

non-zero  Volume ramp is stopped

See also:  UltraRampVolume,UltraRampLinearVolume

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 97

UltraWaitDramDma

3.5.55  UltraWaitDramDma

_____________________________________________________________

Function:  Wait for a DRAM DMA transfer to complete.

Syntax:    void UltraWaitDramDma()

Prototype: gf1proto.h

Remarks:   If a DMA transfer to/from DRAM is started but

told  not  to  wait  for  it  to  complete,  this

function can be used to let the application  wait

for it to complete.

Returns:   None

See also:  UltraDownload,UltraUpload

p                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 98

UltraWaitRecordDma

3.5.56  UltraWaitRecordDma

_____________________________________________________________

Function:  Wait for a Recording DMA transfer to complete.

Syntax:    void UltraWaitRecordDma()

Prototype: gf1proto.h

Remarks:   This can be used to let an application wait until

a complete sample is finished being acquired.

Returns:   None

See also:  UltraRecordData

o                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound             Page 99

UltraWaitRecordDma

3.6  Level 1 Interface Functions

3.6.1  UltraAllocVoice

_____________________________________________________________

Function:  Allocate a voice to use.

Syntax:    int UltraAllocVoice(voice_num,new_num)

int voice_num;

int *new_num;

Prototype: gf1proto.h

Remarks:   This function will return a voice for your

application  to  use.   If  you  supply  a  voice

number,  it  will  attempt   to   allocate   that

particular voice.  If you pass a -1 for the voice

number, it will return the next free voice.  This

function will only allocate voices up to the # of

active  voices  specified  in   the   UltraOpen()

function.  The voice number will be  returned  in

new_num.

Returns:   ULTRA_OK  Got a voice

VOICE_NOT_FREE       Can't get the voice or none left.

VOICE_OUT_OF_RANGE   Specified voice out of range.

See also:  UltraClearVoices, UltraFreeVoices

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 100

UltraClearVoices

3.6.2  UltraClearVoices

_____________________________________________________________

Function:  Reset all voice to un-allocated.

Syntax:    void UltraClearVoices()

Prototype: gf1proto.h

Remarks:   This function will un-allocate all previously

allocated voices.  It would be advisable to  call

this before  using  either  UltraAllocVoice()  or

UltraFreeVoice().

Returns:   None

See also:  UltraAllocVoice, UltraFreeVoice.

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 101

UltraFreeVoice

3.6.3  UltraFreeVoice

_____________________________________________________________

Function:  Free up an allocated voice.

Syntax:    void UltraFreeVoice(voice_num)

int voice_num;

Prototype: gf1proto.h

Remarks:   This function will free up a previously allocated

voice.  This should be used when your application

no  longer  needs  the  voice  so   it   can   be

re-allocated at another time.

Returns:   None

See also:  UltraClearVoices, UltraAllocVoice.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 102

UltraVoiceOff

3.6.4  UltraVoiceOff

_____________________________________________________________

Function:  Turn a voice off & let it decay.

Syntax:    void UltraVoiceOff(voice,end)

int voice;

int end;

Prototype: gf1proto.h

Remarks:   This function will either stop a voice

immediately or let it finish  its  current  loop.

if  'end'  is  equal  to  0  then  it  will  stop

abruptly, else it will finish the  current  loop.

UltraReadVoice() could be  called  after  if  you

need to know when the  loop  finished.   If  used

with UltraSetVoiceEnd(), you could have a sampled

decay on the end  of  your  sample.   This  would

occur if your loop point was not at  the  end  of

your data, and you changed the end point  to  the

real end point of your data and then called  this

function with end not equal 0.

Returns:   Nothing

See also:  UltraReadVoice,UltraSetVoiceEnd

a                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 103

UltraVoiceOn

3.6.5  UltraVoiceOn

_____________________________________________________________

Function:  Turn a voice on at a given frequency.

Syntax:

void UltraVoiceOn(voice,begin,s_loop,e_loop,control,freq)

int voice;

unsigned long begin;          /* initial start location */

unsigned long s_loop;         /* start of loop */

unsigned long e_loop;         /* end of loop */

unsigned char control;

unsigned long freq;

Prototype: gf1proto.h

Remarks:   This function just sets the frequency and

calls UltraStartVoice().

Returns:   None

See also:  UltraSetFrequency,UltraStartVoice

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 104

UltraSetLinearVolume

3.6.6  UltraSetLinearVolume

_____________________________________________________________

Function:  Set the volume to a linearized value.

Syntax:    void UltraSetLinearVolume(voice,index)

int voice;

int index;

Prototype: gf1proto.h

Remarks:   This function indexes into a table to

translate a 0-511 value linear volume to a 0-4095

logarithmic volume.

Returns:   None

See also:  UltraSetVolume

l                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 105

UltraRampLinearVolume

3.6.7  UltraRampLinearVolume

_____________________________________________________________

Function:  Ramp between linear volume settings

Syntax:

void UltraRampLinearVolume(voice,start,end,msecs,mode)

int voice;

unsigned int start;  /* start linear vol (0-511) */

unsigned int end;    /* end linear vol (0-511) */

unsigned long msecs; /* # of millisecs to run ramp ...  */

unsigned char mode;  /* mode to run the vol ramp in ...  */

Prototype: gf1proto.h

Remarks:   This function is used to ramp between

2 linear  volume  settings.   It  uses  the  same

method as UltraSetLinearVolume() to determine the

actual volume settings to use.

Returns:   None

See also:  UltraRampVolume,UltraSetLinearVolume

m                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 106

UltraVectorLinearVolume

3.6.8  UltraVectorLinearVolume

_____________________________________________________________

Function:  Ramp a linear volume from here to end point.

Syntax:    void UltraVectorLinearVolume(voice,end,rate,mode)

int voice;

unsigned int end; /* end ramp volume */

unsigned char rate;

unsigned char mode; /* mode to run the vol ramp in ...  */

Prototype: gf1proto.h

Remarks:   This function can be used to ramp from an

unknown place to a new place.  It  is  useful  if

you  are  doing  volume  envelopes  and  need  to

restart the attack/decay sequence  at  any  time.

This  uses  linear  volume  entries  rather  than

logarithmic.  It can also produce a smoother ramp

from one volume to another.  Arbitrarily  setting

a volume that is far away from the current volume

can cause 'pops'.

Returns:   None

See also:  UltraSetLinearVolume.

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 107

Chapter 4 - Focal Point 3D Sound

Three dimensional audio on the Ultrasound is achieved  by  a

technique called binaural representation.   Basically,  this

means that a mono sound is 'shaped' in such a way that  when

it is presented to the right and  left  ears  properly,  the

sound seems to come from the proper  place  in  space.   The

technique of shaping the sound is called convolution.   This

is done thru algorithms  developed  by  Focal  Point(tm)  3D

Audio.  Focal Point has provided a utility (called fp3d.exe)

to convert a mono sound file to a 3D file capable  of  being

played on an UltraSound.  The basic concept is that the mono

sound is processed in  such  a  way  that  the  output  file

contains up to 6 tracks of sound.  When the sound is  played

back, the volumes and balances  are  adjusted  to  make  the

sound appear to originate from anywhere in the 3D space.  It

is possible to create files that only have 4 tracks and  can

therefore only be positioned in 2 dimensions.  This  may  be

adequate  for  many  applications  and  it  will  make   the

resultant file smaller.

There are 2 types of 3D sound that an application might want

to use.  The first is a sound effect.  This is a sound  that

can be completely loaded into DRAM and played and positioned

at any time.  It is used for things like gunshots, cars etc.

These can be looped sounds or 'one shots'.  The  other  type

of sound is a sound track.  This is for a  very  long  sound

that  cannot  be  loaded  in  DRAM   all   at   once.    The

implementation of each of these methods is  very  different.

A sound effect is implemented using a blocked  data  format.

This means that each track's data is in a block of its  own.

A sound track uses interleaved data.  This  means  that  all

the track's data is interleaved together.   For  example,  a

blocked file would look like this:

HEADER FFFFFFFFFFRRRRRRRRRRBBBBBBBBBBLLLLLLLLLL

An interleaved file would look like this:

HEADER FRBLFRBLFRBLFRBLFRBLFRBLFRBLFRBLFRBLFRBL

F - Front Track

R - Right Track

B - Behind Track

L - Left Track

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 108

Chapter 4 - Focal Point 3D Sound

U - Up Track

D - Down Track

Each of these methods has its advantages and  disadvantages.

First, the interleaved method makes it very easy to read  in

the data in a continuous stream since it will look the  same

all the way through the file.  It is also faster to read 60K

of data once rather than 6 10K reads.  This  makes  it  very

useful for a sound track.  However, one draw back is that it

has a very limited number of frequencies that  it  will  run

at.  The reason for this is rather difficult to explain, but

it pertains to getting the UltraSound's voices to play every

4th (or 5th, etc) sample.   The  UltraSound  normally  would

interpolate between data points but it can't  do  that  here

because the adjacent data points are not in the same  track.

If you want to run at 22050 Hz, then you need to  have  only

28 active voices to accomplish this.  If you want to play  a

track at 44100 Hz, you must  have  only  14  active  voices.

Blocked data doesn't have this problem.  Since the  data  is

NOT interleaved, but in a  block  of  contiguous  DRAM,  its

frequency can be adjusted to any  value.   This  is  usually

very useful for sound effects (rev'ing engines etc).   Also,

since  the  data  is  blocked,  each  track's  data  can  be

allocated separately and can therefore be up  to  256K.   An

interleaved file must fit into 1 256K bank since  we  cannot

play 16bit data across a 256K bank.  See the two example  3D

programs provided (play3d.c play3di.c) for ideas on  how  to

implement 3D sound into you application.

4.1  Creating 3D file

Please read the readme.3d file for this info.

4.2  3D Sound Functions

These are the functions currently available to playback a 3D

sound file.

4.2.1  UltraAbsPosition

_____________________________________________________________

Function:  Place a sound in 3D space (Cartesian).

Syntax:    void UltraAbsPosition3d(sound,xpos,ypos,zpos)

SOUND_3D *sound; /* Handle for the 3D sound */

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 109

UltraAbsPosition

int xpos; /* -511 (left) to +511 (right) */

int ypos; /* -511 (below) to +511 (above) */

int zpos; /* -511 (behind) to +511 (ahead) */

Prototype: threed.h

Remarks:   This function will position a sound using

standard cartesian coordinates.  The  X  position

is right/left.  The Y position is up/down.  The Z

position is ahead/behind.  X and Z determine  the

azimuth position and Y determines the  elevation.

The distance away from you  is  determined  using

trigonometry  .   Once  these   are   determined,

UltraAngPosition3d() is called.  Be careful  when

using this function,  since  the  trig  functions

will be needed from  the  C  libraries.   If  the

distance is calculated to be greater than 511, it

is clipped to 511.   Also,  if  the  distance  is

calculated to be 0, no positioning is done  since

the origin is undefined.  (i.e.  A  sound  cannot

be generated INSIDE your head (origin)).

Returns:   None

See also:  UltraAngPosition3d

.                      Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 110

UltraAngPosition3d

4.2.2  UltraAngPosition3d

_____________________________________________________________

Function:  Position a sound using polar coordinates

Syntax:

void UltraAngPosition3d(sound,azimuth,elevation,volume)

SOUND_3D *sound;/* Handle */

int azimuth; /* Horiz plane (-180 to +180 deg) */

int elevation; /* Vert plane (-90 to +90 deg) */

int volume; /* volume level at this position */

Prototype: threed.h

Remarks:   This function will position a 3D sound using

polar coordinates.  The azimuth and elevation  is

specified in degrees.  Azimuth is  the  angle  in

the  horizontal  plane.   Straight  ahead  is   0

degrees, 90 degrees is to the right, -90  degrees

is to the left and  180  (or  -180)  is  directly

behind you.  If  an  angle  larger  than  180  or

smaller that -180 is specified, it  is  converted

to its -180 to 180 equivalent.  For example,  270

degrees is equivalent to -90 degrees.   Elevation

is the angle of  elevation  above  or  below  the

horizontal plane.  0 degrees is no elevation,  90

degrees is straight up, and -90 is straight down.

Any angle larger than 90 or smaller that  -90  is

ignored.

If    more    precision    is     needed,     use

UltraAngFltPosition3D() so  that  floating  point

arithmetic is used.   Since  that  function  uses

floating point functions,  your  application  may

not want to use it.

Returns:   None

See also:  UltraAbsPosition3d

n                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 111

UltraAngFltPosition3d

4.2.3  UltraAngFltPosition3d

_____________________________________________________________

Function:  Position a sound using polar coordinates.

Syntax:

void UltraAngFltPosition3d(sound,azimuth,elevation,volume)

SOUND_3D *sound; /* Handle */

double azimuth; /* Horiz plane (-180.0 to +180.0 deg) */

double elevation; /* Vert plane (-90.0 to +90.0 deg) */

int volume; /* volume level at this position */

Prototype: threed.h

Remarks:   This function will position a 3D sound using

polar coordinates.  The azimuth and elevation  is

specified  in  as  a  floating  point  number  in

degrees.  For example, 45 and  one  half  degrees

would be specified as 45.5.  Azimuth is the angle

in the horizontal plane.   Straight  ahead  is  0

degrees, 90 degrees is to the right, -90  degrees

is to the left and  180  (or  -180)  is  directly

behind you.  If  an  angle  larger  than  180  or

smaller that -180 is specified, it  is  converted

to its -180 to 180 equivalent.  For example,  270

degrees is equivalent to -90 degrees.   Elevation

is the angle of  elevation  above  or  below  the

horizontal plane.  0 degrees is no elevation,  90

degrees is straight up, and -90 is straight down.

Any angle larger than 90 or smaller that  -90  is

ignored.

If your application does not wish  (or  need)  to

use   the   floating    point    function,    use

UltraAngPosition().

i                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 112

UltraLoad3dEffect

4.2.4  UltraLoad3dEffect

_____________________________________________________________

Function:  Load a 3D effect into DRAM

Syntax:    int UltraLoad3dEffect(sound,filename,pc_buffer,size)

SOUND_3D *sound;

char *filename;

void far *pc_buffer;

unsigned int size;

Prototype: threed.h

Remarks:   This function is used to load a 3D sound into the

DRAM on the UltraSound.   It  will  allocate  all

necessary resources so that UltraStart3D() can be

called later.  As many buffers of DRAM as  needed

will be allocated along with the proper number of

voices.  These resources will be  freed  up  when

you  call  UltraUnLoad3dEffect().   The  filename

MUST be a properly formatted  file.   This  means

that it must have the proper header and  3D  data

in it.  The pc_buffer is  a  buffer  supplied  by

your  application  for  this  functions  use   to

download data into the  UltraSound.   The  'size'

parameter is the  size  of  the  pc_buffer.   The

larger  this  buffer  is,  the  faster  it   will

download.

Returns:   ULTRA_OK             no error

NO_3D_FILE           3D file not found.

BAD_3D_HDR           3D header is bad.

NO_3D_HDR            File doesn't have a valid header.

NOT_BLOCK_DATA       File not in blocked format.  (interleaved)

NO_FREE_VOICES       Not enough free voices

BAD_FILE_DATA        Not enough data.  Hdr wrong or bad file.

CORRUPT_MEM          Memory structs have been corrupted

DMA_BUSY             Dma channel busy.

See also:  UltraMemAlloc,UltraVoiceAlloc,UltraDownload

s                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 113

UltraSetFreq3D

4.2.5  UltraSetFreq3D

_____________________________________________________________

Function:  Set Frequency of 3D sound

Syntax:    void UltraSetFreq3D(sound,frequency)

SOUND_3D *sound;

unsigned long frequency;

Prototype: threed.h

Remarks:   This function will allow you to alter the frequency

that the 3D sound is using.  This allows  you  to

do pitch shifting to get  a  doppler  shift  type

effect.   This  function  can  only  be  done  on

blocked data, not interleaved.

Returns:   ULTRA_OK  no problem

NOT_BLOCKED_DATA     Can't change freq of interleaved data.

See also:

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 114

UltraRelease3dInterleave

4.2.6  UltraRelease3dInterleave

_____________________________________________________________

Function:  Release Interleaved 3D resources

Syntax:    void UltraRelease3dInterleave(sound)

SOUND_3D *sound;

Prototype: threed.h

Remarks:   This functions will release the DRAM and

voices  allocated  for  this  interleaved   sound

track.

Returns:   None

See also:  UltraSetup3dInterleave

r                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 115

UltraSetup3dInterleave

4.2.7  UltraSetup3dInterleave

_____________________________________________________________

Function:  Setup for an interleaved 3D sound

Syntax:    int UltraSetup3dInterleave(sound,filename,size)

SOUND_3D *sound;

char *filename;

unsigned long size;

Prototype: threed.h

Remarks:   This function will allocate the voices and memory

necessary to playback an  interleaved  3D  sound.

It will NOT load any of the file data into  DRAM.

Your application is  responsible  for  that.   It

will open  the  file  and  read  the  header  and

allocate the appropriate resources and setup  the

begin,start and end loop points for  each  voice.

Since they are interleaved, the loop  points  are

staggared appropriately.

Returns:   ULTRA_OK             no error

NO_3D_FILE           3D file not found.

BAD_3D_HDR           3D header is bad.

NO_3D_HDR            File doesn't have a valid header.

NOT_INTERLEAVED_DATA File in blocked format.

NO_FREE_VOICES       Not enough free voices

BAD_FILE_DATA        Not enough data.  Hdr wrong or bad file.

CORRUPT_MEM          Memory structs have been corrupted

See also:  UltraLoad3dEffect

r                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 116

UltraStart3d

4.2.8  UltraStart3d

_____________________________________________________________

Function:  Start a 3D sound.

Syntax:    void UltraStart3d(sound)

SOUND_3D *sound;

Prototype: threed.h

Remarks:   This function will start a 3D sound.  If you want

it to begin at a specific point in space, be sure

that you position it first.  It is not  necessary

to stop the sound before starting it again.

Returns:   None

See also:  UltraStart3d

s                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 117

UltraStop3d

4.2.9  UltraStop3d

_____________________________________________________________

Function:  Stop a 3d sound.

Syntax:    void UltraStop3D(sound,abruptly)

SOUND_3D *sound;

int abruptly;

Prototype: threed.h

Remarks:   This function will stop a 3D sound.  There are

two ways to stop a sound.  If the 'abruptly' flag

is  TRUE,  then   the   sound   will   shut   off

immediately.  If it is FALSE, then the sound will

be ramped down very quickly.  The  second  method

will give a smoother transition.

Returns:   None

See also:

T                     Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 118

UltraUnLoad3dEffect

4.2.10  UltraUnLoad3dEffect

_____________________________________________________________

Function:  Free up a 3D effect resources.

Syntax:    void UltraUnLoad3dEffect(sound)

SOUND_3D *sound;

Prototype: threed.h

Remarks:   This function is used to free up all the

resources (voices & DRAM) that a 3D  sound  uses.

Since a 3D sound can use a lot of  the  voices  &

DRAM, you should free up the  resources  whenever

you can.

Returns:   None

See also:  UltraLoad3dEffect

s                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 119

Appendix A - Error Codes

These are defined in ultraerr.h

#define ULTRA_OK		  1   No error

#define BAD_NUM_OF_VOICES  2   must be 14-32

#define NO_MEMORY	  3   Not enough free DRAM left

#define CORRUPT_MEM	  4   memory structures are corrupt

#define NO_ULTRA		  5   Can't find an UltraSound

#define DMA_BUSY		  6   This DMA channel is still busy

#define BAD_DMA_ADDR	  7   auto init across page boundaries

#define VOICE_OUT_OF_RANGE 8   allocate a voice past # active

#define VOICE_NOT_FREE	  9   voice has already been allocated

#define NO_FREE_VOICES	  10  not any voices free

Error codes for 3D functions. These are defined in threed.h.

#define NO_3D_FILE	101   Can't open file

#define BAD_3D_HDR	102   Header for file is corrupt/non-existent

#define NO_DRAM_3D	103   Not enough room for this sound

#define NO_3D_HDR	104   No header on this file

#define NOT_BLOCK_DATA	105   data not in block format

#define BAD_FILE_DATA	106   not enough data as header says

#define NOT_INTERLEAVED_DATA	107	data not in interleaved format

#define BAD_3D_FREQ		108	invalid freq. for interleaved 3D data

v                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 120

Appendix B - Volume Control Bits

Here are the volume ramp control bit definitions:

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Reserved

|   |   |   |   |   |   +-------- Reserved

|   |   |   |   |   +------------ Reserved

|   |   |   |   +---------------- loop enable (0=no loop, 1=loop)

|   |   |   +-------------------- bi-direction. loop (1=enable)

|   |   +------------------------ Enable volume ramp IRQ

|   +---------------------------- Reserved

+-------------------------------- Reserved

This  specifies  the  bits  that  should  be  set   by   the

application to get a particular type of volume  ramp.   This

should    be    supplied    to     UltraRampVolume()     and

UltraRampLinearVolume().  If IRQ's are  enabled,  make  sure

that  you  have  set  up  a   volume   IRQ   handler.    See

UltraVolumeHandler().  This can be used to create  your  own

multi-point volume envelopes.

Bi-directional looping can  be  used  to  create  a  vibrato

effect.

a                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 121

Appendix C - Voice Control Bits

Here are the voice control bit definitions:

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Reserved

|   |   |   |   |   |   +-------- Reserved

|   |   |   |   |   +------------ data type (0=8 bit 1=16 bit)

|   |   |   |   +---------------- loop enable (0=no loop, 1=loop)

|   |   |   +-------------------- bi-direction. loop (1=enable)

|   |   +------------------------ Enable wavetable IRQ

|   +---------------------------- Reserved

+-------------------------------- Reserved

The UltraSound is capable of playing back 8 or 16 bit  data.

(It can only record 8 bit).  Stereo is handled  by  using  2

voices.  It can loop on the data in either a uni-directional

or bi-directional mode.  If you have asked that  this  voice

generate a wavetable IRQ when it hits the end  of  the  data

(or loop point, if looping is specified), be sure  you  have

specified  a  wavetable  irq  hander.   (UltraWaveHander()).

These  mode  bits  would  be  passed  to   UltraStartVoice()

,UltraSetLoopMode() and UltraNoteOn().

b                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 122

Appendix D - DMA Control Bits

Here are the dma to/from DRAM control bit definitions:

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Reserved

|   |   |   |   |   |   +-------- 0=write,1=Read

|   |   |   |   |   +------------ Reserved

|   |   |   |   +---------------- Reserved

|   |   |   +-------------------- Reserved

|   |   +------------------------ Reserved

|   +---------------------------- Data size (0=8 bit, 1=16 bit)

+-------------------------------- (1=convert to 2's comp.)

Note:

The UltraSound DRAM location MUST be on a 32 byte boundary.

UltraMemAlloc() enforces this stipulation.

This  is  the  definition  of  the  bits  to  be  passed  to

UltraUpload() and UltraDownload().  To DMA the data  out  of

DRAM, use UltraUpload() and to send data to  the  DRAM,  use

UltraDownload().  Be sure to specify  the  sample  size  and

whether or not the data is  in  two  compliment  form.   The

UltraSound  can  only  play  data  back  that  is  in  two's

compliment form.  If your  sample  is  in  one's  compliment

form, turn on bit 7.  The data will be translated  to  two's

compliment as it is being DMA'ed.

NOTE:

If you are poking data into DRAM,  you  MUST  put

the data in twos compliment  yourself.   This  is

accomplished by exclusive  or-ing  the  high  bit

with a 1.

b                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 123

Appendix E - Recording Control Bits

Here are the recording control bit definitions:

=================================

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

=================================

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +---- Reserved

|   |   |   |   |   |   +-------- mode (0=mono,1=stereo)

|   |   |   |   |   +------------ Reserved

|   |   |   |   +---------------- Reserved

|   |   |   +-------------------- Reserved

|   |   +------------------------ Reserved

|   +---------------------------- Reserved

+-------------------------------- Reserved

r                    Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 124

Appendix F - Patch Header

/***************************************************************************

*   NAME:  PATCH.H

**   COPYRIGHT:

**   "Copyright (c) 1991,1992, by FORTE

**

**       "This software is furnished under a license and may be used,

**       copied, or disclosed only in accordance with the terms of such

**       license and with the inclusion of the above copyright notice.

**       This software or any other copies thereof may not be provided or

**       otherwise made available to any other person. No title to and

**       ownership of the software is hereby transferred."

****************************************************************************

*  CREATION DATE: 07/01/92

*--------------------------------------------------------------------------*

*     VERSION   DATE      NAME      DESCRIPTION

*>   1.0   07/01/92      Original

***************************************************************************/

#define ENVELOPES   6

/* This is the definition for what FORTE's patch format is. All .PAT */

/* files will have this format. */

#define HEADER_SIZE   12

#define ID_SIZE       10

#define DESC_SIZE     60

#define RESERVED_SIZE   40

#define PATCH_HEADER_RESERVED_SIZE 36

#define LAYER_RESERVED_SIZE   40

#define PATCH_DATA_RESERVED_SIZE   36

#define GF1_HEADER_TEXT   "GF1PATCH110"

typedef struct

{

char      header[ HEADER_SIZE ];

char      gravis_id[ ID_SIZE ];   /* Id = "ID#000002" */

char      description[ DESC_SIZE ];

unsigned char   instruments;

char      voices;

char      channels;

unsigned int   wave_forms;

unsigned int   master_volume;

unsigned long   data_size;

char      reserved[ PATCH_HEADER_RESERVED_SIZE ];

} PATCHHEADER;

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 125

Appendix F - Patch Header

typedef struct

{

unsigned int   instrument;

char      instrument_name[ 16 ];

long      instrument_size;

char      layers;

char      reserved[ RESERVED_SIZE ];

} INSTRUMENTDATA;

typedef struct

{

char      layer_duplicate;

char      layer;

long      layer_size;

char      samples;

char      reserved[ LAYER_RESERVED_SIZE ];

} LAYERDATA;

typedef struct

{

char      wave_name[7];

unsigned char   fractions;

long      wave_size;

long      start_loop;

long      end_loop;

unsigned int   sample_rate;

long      low_frequency;

long      high_frequency;

long      root_frequency;

int       tune;

unsigned char   balance;

unsigned char   envelope_rate[ ENVELOPES ];

unsigned char   envelope_offset[ ENVELOPES ];

unsigned char   tremolo_sweep;

unsigned char   tremolo_rate;

unsigned char   tremolo_depth;

unsigned char   vibrato_sweep;

unsigned char   vibrato_rate;

unsigned char   vibrato_depth;

/* bit 0 = 8 or 16 bit wave data. */

/* bit 1 = Signed - Unsigned data. */

/* bit 2 = looping enabled-1. */

/* bit 3 = Set is bidirectional looping. */

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 126

Appendix F - Patch Header

/* bit 4 = Set is looping backward. */

/* bit 5 = Turn sustaining on. (Env. pts. 3)*/

/* bit 6 = Enable envelopes - 1 */

char      modes;

int      scale_frequency;

unsigned int   scale_factor;      /* from 0 to 2048 or 0 to 2 */

char     reserved[ PATCH_DATA_RESERVED_SIZE ];

} PATCHDATA;

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 127

Appendix G - 3D File Header

/***************************************************************************

*   NAME:  HDR3D.H

**   COPYRIGHT:

**   "Copyright (c) 1992, by FORTE

**

**       "This software is furnished under a license and may be used,

**       copied, or disclosed only in accordance with the terms of such

**       license and with the inclusion of the above copyright notice.

**       This software or any other copies thereof may not be provided or

**       otherwise made available to any other person. No title to and

**       ownership of the software is hereby transferred."

****************************************************************************

*  CREATION DATE: 02/01/93

*--------------------------------------------------------------------------*

*     VERSION   DATE      NAME      DESCRIPTION

*>   1.0   02/01/93      Original

***************************************************************************/

/* Bit definitions for tracks that are in a 3D file ... */

#define FRONT_TRACK      0x01

#define RIGHT_TRACK      0x02

#define REAR_TRACK       0x04

#define LEFT_TRACK       0x08

#define ABOVE_TRACK      0x10

#define BELOW_TRACK      0x20

#define FBLOCK_3D      0x01      /* File data is blocked, not interleaved */

#define F16BIT_3D      0x02      /* 16 bit data */

#define FTWOS_CMP_3D   0x04      /* sound is in twos complement form */

#define FLOOPED_3D     0x08      /* sound is looped , not one shot*/

#define FBI_LOOP_3D    0x10      /* sound is bi-directional */

#define F3D_TYPE       0x60      /* Type of 3D sound */

/* types (F3D_TYPE) of 3D sound (only binaural currently supported) */

#define F_BINAURAL      0x20      /* Binaural representation */

#define F_SURROUND      0x40      /* Surround sound (???) */

#define F_QSOUND        0x60      /* Q sound (???) */

/* this structure is exactly 256 bytes long ... */

typedef struct {

char id[10];             /* 3D FILE tag */

int major;               /* major version # */

int minor;               /* minor version # */

char description[80];

unsigned int type;       /* See above .... */

Advanced Gravis and Forte Technologies

Low Level Toolkit for Gravis' UltraSound            Page 128

Appendix G - 3D File Header

int tracks;              /* tracks included in this file ... */

int reserve1[24];        /* for expansion */

int maxvol;              /* volume can range from 0 to maxvol */

int reserve2[10];        /* for expansion */

unsigned long blocksize;   /* # of bytes in block data */

unsigned long loop_offset; /* offset to where loop begins (bytes) ... */

unsigned long reserve3[9]; /* for expansion */

unsigned long frequency;   /* initial playback frequency */

unsigned long reserve4[10];/* for expansion */

} FILEHDR_3D;

Advanced Gravis and Forte Technologies