Small doc on using SVGA with DOS32.

1. Check if the videomode we want is supported on the computer.

The problem here is that we need some low memory, where we can let the VBE

put some info about the mode.  This is solved, however, by using the 8k

buffer that DOS32 uses for file I/O.

(We'll make a near ptr to the videomemory when we're at function 0ee02h

anyway.)

--->8----- API.DOC ----------

GET DOS32 ADDRESS INFORMATION            V3.01+

IN:    AX = EE02h

OUT:   EBX = 32bit linear address of the program segment

EDX = Total size in bytes of the programs .EXE file after

linking.

ESI = Offset address of PSP ( Program Segment Prefix )

EDI = Offset address of Program Environment

ECX = Offset address of the programs .EXE file name and

path (in ASCIZ format, i.e. terminates with a zero)

AX = Real mode segment value of the 8Kb buffer that is

used by the file read/write services.

---8<----- API.DOC ----------

The values we need here are AX and EBX, we need to make a near pointer out

of that realmode segment value.

lowmemptr       dd      ?

lowmemseg       dw      ?

linearadr       dd      ?

videomem        dd      ?

mov     eax,0ee02h

int     31h

mov     lowmemseg,ax

mov     linearadr,ebx

push    ax      ;\

xor     eax,eax ; > Clears the high bits of eax

pop     ax      ;/

shl     eax,4   ; Makes eax the absolute address of the RM segment

sub     eax,ebx ; Makes eax a near pointer to the RM segment

mov     lowmemptr,eax

mov     eax,0a0000h

sub     eax,ebx

mov     videomem,eax

Now when we have our segment, and accompanying near pointer, we can make a

real mode callback to the VBE.

--->8----- API.DOC ----------

...expects a pointer to a real

mode call structure whish is used to passing all registers to and

from the real mode switch. The format of the structure is defined

as:                      Offset     Register

00h         EDI

04h         ESI

08h         EBP

0Ch  Reserved by system

10h         EBX

14h         EDX

18h         ECX

1Ch         EAX

20h         Flags

22h         ES

24h         DS

26h         FS

28h         GS

2Ah         IP

2Ch         CS

2Eh         SP

30h         SS

Simulate a Real Mode interrupt            V3.00+

IN:

AX = 0300h

BL = Interrupt number

BH = Flags

Bit 0  = 1 resets the interrupt controller and A20

line. Other flags reserved and must be 0

CX =  Number of  words to  copy from  protected mode to

real mode stack

ES:EDI = Selector:Offset of real mode call structure

OUT:

If function was successful:

Carry flag is clear.

ES:(E)DI =  Selector:Offset of  modified real mode call

structure

If function was not successful:

Carry flag is set.

---8<----- API.DOC ----------

So, we need a structure defined as described in API.DOC:

--->8----- GFX.ASM ----------

_	label

_edi	label	dword

_di	label	word

_dih	db	?

_dil	db	?

_edih	db	?

_edil	db	?

_esi	label	dword

_si	label	word

_sih	db	?

_sil	db	?

_esih	db	?

_esil	db	?

_ebp	label	dword

_bp	label	word

_bph	db	?

_bpl	db	?

_ebph	db	?

_ebpl	db	?

_reserv	dd	?

_ebx	label	dword

_bx	label	word

_bh	db	?

_bl	db	?

_ebh	db	?

_ebl	db	?

_edx	label	dword

_dx	label	word

_dh	db	?

_dl	db	?

_edh	db	?

_edl	db	?

_ecx	label	dword

_cx	label	word

_ch	db	?

_cl	db	?

_ech	db	?

_ecl	db	?

_eax	label	dword

_ax	label	word

_ah	db	?

_al	db	?

_eah	db	?

_eal	db	?

_flags	dw	?

_es	dw	?

_ds	dw	?

_fs	dw	?

_gs	dw	?

_ip	dw	?

_cs	dw	?

_sp	dw	?

_ss	dw	0

---8<----- GFX.ASM ----------

--->8----- INTERRUP.LST ----------

--------V-104F00-----------------------------

INT 10 - VESA SuperVGA BIOS (VBE) - GET SuperVGA INFORMATION

AX = 4F00h

ES:DI -> buffer for SuperVGA information (see #0063)

Return: AL = 4Fh if function supported

AH = status

00h successful

ES:DI buffer filled

01h failed

---VBE v2.0---

02h function not supported by current hardware configuration

03h function invalid in current video mode

Desc:   determine whether VESA BIOS extensions are present and the capabilities

supported by the display adapter

SeeAlso: AX=4E00h,AX=4F01h,AX=7F00h,AX=A00Ch

Index:  installation check;VESA SuperVGA

Format of SuperVGA information:

Offset  Size    Description     (Table 0063)

00h  4 BYTEs   (ret) signature ("VESA")

(call) VESA 2.0 request signature ("VBE2"), required to receive

version 2.0 info

04h    WORD    VESA version number (one-digit minor version)

06h    DWORD   pointer to OEM name

"761295520" for ATI

0Ah    DWORD   capabilities flags (see #0064)

0Eh    DWORD   pointer to list of supported VESA and OEM video modes

(list of words terminated with FFFFh)

12h    WORD    total amount of video memory in 64K blocks

---VBE v1.x ---

14h 236 BYTEs  reserved

---VBE v2.0 ---

14h    WORD    OEM software version

16h    DWORD   pointer to vendor name

1Ah    DWORD   pointer to product name

1Eh    DWORD   pointer to product revision string

22h 222 BYTEs  reserved

100h 256 BYTEs  OEM scratchpad

Notes:  the list of supported video modes is stored in the reserved portion of

the SuperVGA information record by some implementations, and it may

thus be necessary to either copy the mode list or use a different

buffer for all subsequent VESA calls

the 1.1 VESA document specifies 242 reserved bytes at the end, so the

buffer should be 262 bytes to ensure that it is not overrun; for

v2.0, the buffer should be 512 bytes

the S3 specific video modes will most likely follow the FFFFh

terminator at the end of the standard modes.  A search must then

be made to find them, FFFFh will also terminate this second list

Bitfields for VESA capabilities:

Bit(s)  Description     (Table 0064)

0      DAC can be switched into 8-bit mode

1      non-VGA controller

2      programmed DAC with blank bit

3-31   reserved

---8<----- INTERRUP.LST ----------

Now, we need to set up the values to do the callback:

mov     ax,lowmemseg

mov     _es,ax

mov     _di,0

mov     _ax,4f00h

mov     eax,300h

mov     ebx,10h

xor     ecx,ecx

lea     edi,_           ;our RM callback structure

int     31h

If all went well, (ah=00h, al=4fh), we should now have a realmode seg:ofs ptr

to a word list of supported video modes.  Make it a near ptr and scan it to

see if what we want is available:

cmp     _ax,4fh

jne     vbe_error

mov     esi,lowmemptr

xor     eax,eax

xor     ebx,ebx

mov     ax,word ptr esi[0eh]

mov     bx,word ptr esi[10h]

shl     ebx,4

add     eax,ebx

sub     eax,linearadr

mov     esi,eax

mov     bx,101h         ;desired video mode

loop1:

lodsw

cmp     ax,-1

je      mode_not_supported

cmp     ax,bx

jne     loop1

We've got our video mode! :)

2. We need the window granularity to calculate number of banks making up 64k

This means interfacing the VBE, and using the RM buffer again.

--->8----- INTERRUP.LST ----------

--------V-104F01-----------------------------

INT 10 - VESA SuperVGA BIOS - GET SuperVGA MODE INFORMATION

AX = 4F01h

CX = SuperVGA video mode

ES:DI -> 256-byte buffer for mode information (see #0065)

Return: AL = 4Fh if function supported

AH = status

00h successful

ES:DI buffer filled

01h failed

Desc:	determine the attributes of the specified video mode

SeeAlso: AX=4F00h,AX=4F02h

Format of VESA SuperVGA mode information:

Offset	Size	Description	(Table 0065)

00h	WORD	mode attributes (see #0066)

02h	BYTE	window attributes, window A (see #0067)

03h	BYTE	window attributes, window B (see #0067)

04h	WORD	window granularity in KB

06h	WORD	window size in KB

08h	WORD	start segment of window A

0Ah	WORD	start segment of window B

0Ch	DWORD	-> FAR window positioning function (equivalent to AX=4F05h)

10h	WORD	bytes per scan line

---remainder is optional for VESA modes in v1.0/1.1, needed for OEM modes---

12h	WORD	width in pixels (graphics) or characters (text)

14h	WORD	height in pixels (graphics) or characters (text)

16h	BYTE	width of character cell in pixels

17h	BYTE	height of character cell in pixels

18h	BYTE	number of memory planes

19h	BYTE	number of bits per pixel

1Ah	BYTE	number of banks

1Bh	BYTE	memory model type (see #0068)

1Ch	BYTE	size of bank in KB

1Dh	BYTE	number of image pages

1Eh	BYTE	reserved (0)

---VBE v1.2+---

1Fh	BYTE	red mask size

20h	BYTE	red field position

21h	BYTE	green mask size

22h	BYTE	green field size

23h	BYTE	blue mask size

24h	BYTE	blue field size

25h	BYTE	reserved mask size

26h	BYTE	reserved mask position

27h	BYTE	direct color mode info

bit 0: color ramp is programmable

bit 1: bytes in reserved field may be used by application

---VBE v2.0 ---

28h	DWORD	physical address of linear video buffer

2Ch	DWORD	pointer to start of offscreen memory

30h	WORD	KB of offscreen memory

32h 206 BYTEs	reserved (0)

Bitfields for VESA SuperVGA mode attributes:

Bit(s)	Description	(Table 0066)

0	mode supported

1	optional information available

2	BIOS output supported

3	set if color, clear if monochrome

4	set if graphics mode, clear if text mode

---VBE v2.0 ---

5	mode is not VGA-compatible

6	bank-switched mode not supported

7	linear framebuffer mode supported

Bitfields for VESA SuperVGA window attributes:

Bit(s)	Description	(Table 0067)

0	exists

1	readable

2	writable

3-7	reserved

(Table 0068)

Values for VESA SuperVGA memory model type:

00h	text

01h	CGA graphics

02h	HGC graphics

03h	16-color (EGA) graphics

04h	packed pixel graphics

05h	"sequ 256" (non-chain 4) graphics

06h	direct color (HiColor, 24-bit color)

07h	YUV (luminance-chrominance, also called YIQ)

08h-0Fh reserved for VESA

10h-FFh OEM memory models

---8<----- INTERRUP.LST ----------

mov     ax,lowmemseg

mov     _es,ax

mov     _di,0

mov     _ax,4f01h

mov     _cx,101h        ;video mode

mov     eax,300h

mov     ebx,10h

xor     ecx,ecx

lea     edi,_           ;our RM callback structure

int     31h

If all is OK, we will have the wgran parameter at offset 4 in the RM

segment.  Since we have 64k of usable video memory we need to know how

many banks we should add each time we have moved 64k of data.  Many thanks

to Per Almered of Excellence in Art '97, for pointing this obvious and

quite significant optimization out to me. :)  (a difference between 4 and

74 interrupts per frame), this way we will always do 4 ints.

The number of banks we should add to get 64k is calculated by mirroring

the 7 bit-unsigned granularity-size in kb. (or dividing 64 with wgran, but

somehow i dislike unneccessary divisions. =)

bank size    wgran              banks needed to get 64k

1024         1 (0000001b)  ->  64 (1000000b)

2048         2 (0000010b)  ->  32 (0100000b)

4096         4 (0000100b)  ->  16 (0010000b)

8192         8 (0001000b)  ->   8 (0001000b)

16384        16 (0010000b)  ->   4 (0000100b)

32768        32 (0100000b)  ->   2 (0000010b)

65536        64 (1000000b)  ->   1 (0000001b)

deltabank       dd      ?

cmp     _ax,4fh

jne     vbe_error

mov     esi,lowmemptr

movzx   ebx,word ptr esi[4]

bsf     ecx,ebx

mov     ebx,40h

shr     ebx,cl

mov     deltabank,ebx

We've got deltabank. :)

3. Allocate memory for our virtual screen and switch to the SVGA mode.

This is really simple to do.

--->8----- API.DOC ----------

ALLOCATE A MEMORY BLOCK               V3.00+

The entire application code,data and stack is initially

contained in one large memory block. This service here is used for

allocating extra memory blocks for the application.

IN:  AX = EE42h

EDX = Size of the memory block requested to allocate in

bytes.

OUT:  EAX = The actual allocated size of the memory block in bytes

EDX = Near pointer to the base address of the block relative

to the main program segment.

If the returned size was less than the requested size then

the Carry flag is set otherwise the carry flag is cleared.

Notes:

o  The value expected in EDX will be rounded off to the next

4KB boundary.  Example, allocating 51001h bytes will actually

allocate 52000h bytes.

o  Never call this function in a interrupt handler or after

using the terminated and stay resident function ( AX=EE30h ).

o  If EAX is returned with zero then no memory was allocated and

contents of EDX are undefined.

o  The function will fail if the requested size is zero.

o  No more than 64 allocations can be made. If the application

requires a faster more versatile memory management then

please use Peter Anderson's memory sub-functions from PAL

library. See example files on how to use. This function here

is should used a minimum number of times.

o  When not running under a DPMI server ( i.e  Raw,XMS or VCPI )

then extended memory is allocated before conventional memory

is. For example, if there is no more extended memory free

then conventional memory will be allocated. In either case

DOS32 sets up the 386 page tables such that the allocated

memory is addressed in a straight linear block when it may

actually be physically scattered throughout RAM.

---8<----- API.DOC ----------

vscreen dd      ?

mov     eax,0ee42h

mov     edx,640*480

int     31h

jc      not_enough_memory

mov     vscreen,edx

mov     eax,4f02h

mov     ebx,101h        ;our video mode..

int     10h

Done. :)

4. Put something on the virtual screen.

5. Flip the virtual screen out to video memory.

This is not the best way to do it, but it's fairly simple to understand.

flip	proc

push	eax ebx ecx edx esi edi ebp

xor	edx,edx		;bank counter

xor	ebx,ebx		;used to clear the buffer

call	setbank

mov	ebp,640*480	;bytes left to move-counter

mov	esi,vscreen

outer_loop:

mov	edi,videomem

mov	ecx,ebp

cmp	ebp,65536

jb	loop_start

mov	ecx,65536

loop_start:

and	ecx,not 11b

sub	ebp,ecx

loop_label1:

mov	eax,[esi]	;read a dword from the buffer

mov	[edi],eax	;and write it to video mem

mov	[esi],ebx	;clear the dword in the buffer

add	esi,4		;update pointers

add	edi,4		;update pointers

sub	ecx,4

jnz	loop_label1	;redo wgranbytes(65536)/4 times

or	ebp,ebp		;are we done?

jz	flpbye		;yes, then quit..

add	edx,deltabank	;increase bank counter

call	setbank

cmp	ebp,3		;check if we have more than 3 bytes left,

ja	outer_loop	;if so then jump up

mov	ecx,ebp		;otherwise, move these bytes to video mem

loop2_start:	;this loop is used to move the bytes left

mov	al,[esi]	;read one byte from buffer

mov	[edi],al	;and write to video mem

mov	[esi],bl	;clear buffer

inc	esi		;increase source pointer

inc	edi		;increase destination pointer

dec	ecx

jnz	loop2_start	;play it again, Sam!

flpbye:	pop	ebp edi esi edx ecx ebx eax

retn

flip	endp

setbank proc

push    ax bx dx

mov     ax,4f05h

xor     bx,bx

int     10h

pop     dx bx ax

retn

setbank endp

OK! That's all folks.  Check out DOS32VBE.ASM included in the

package, I've put together the different sources listed in this

document, and made a simple filling-routine.  Enjoy, and feel free

to send comments to stuge@kuai.se! :)

-- Peter Stuge of Excellence in Art '97 - NodeRunner of ComeDY